# Viva La Vida — WRO FE Open Challenge

VIVA ΛA VIΔΑ

In a robotics/engineering context, it is a fun, playful slogan meaning “Long live the screw!”, celebrating the simple but essential mechanical component!

- Table of Contents

1	Overview & Scope	1

2	The robot - Hardware & Wiring	1

3	System Architecture	3

4	Decision‑Making Architecture	6

5	Finite State Machine (FSM)	7

6	Parameters & Profiles	8

7	Calibration & Ops	8

8	KPIs & Test Protocol	9

9	Compliance (Rules Snapshot)	9

10	Risks & Mitigations	9

11	Diagrams	10

12	Appendix A — JSON Override Example	11

13	Appendix B – Variables	12

14	Appendix C — Source Code Analysis	16

15	Appendix D - Source code	19

# Overview & Scope

Mission: autonomously complete three laps inside a 3×3 m field containing a randomized 1×1 m inner island that creates corridors. Robot must maintain direction, avoid the walls, and stop safely. Competition rules: single power switch + Start button; no wireless during the run.

# The robot - Hardware & Wiring

## Bill of Materials

| Component | Role | Interface/Notes |
| --- | --- | --- |
| Raspberry Pi | Main controller | GPIO/I²C |
| PCA9685 | Servo + motor PWM | 50 Hz PWM; I²C |
| MPU6050 | Gyro-Z for yaw | I²C |
| Ultrasonic x3 | Front/Left/Right | Trig/Echo via gpiozero |
| VL53L0X (opt.) | ToF sensors | I²C + XSHUT addressing |
| Steering Servo | Single actuator | 1000–2000 µs pulse |
| Motor + Driver/ESC | Propulsion | PWM ch1/ch2 (FWD/REV) |
| Start Button | Headless start | GPIO20 (pull-up; active LOW) |
| Status LEDs | Green ready / Red status | GPIO19 / GPIO13 |
| Battery/Power | 5–6 V servo; 5 V logic | Common ground |

## Pin Map

| Subsystem | Signal | Pin/Channel | Dir | Notes |
| --- | --- | --- | --- | --- |
| Start Button | START_BTN | GPIO 20 | IN | Pull-up; active LOW |
| LEDs | GREEN/RED | GPIO 19 / 13 | OUT | Status/ready |
| US Front | TRIG / ECHO | 22 / 23 | OUT / IN | max_distance 3.43 m |
| US Left | TRIG / ECHO | 27 / 17 | OUT / IN | queue_len 5 |
| US Right | TRIG / ECHO | 5 / 6 | OUT / IN | queue_len 5 |
| Servo | SERVO_CHANNEL | PCA ch 0 | PWM | Steering |
| Motor | MOTOR_FWD / REV | PCA ch 1 / 2 | PWM | Duty 0–100% |
| I²C | SCL / SDA | board.SCL / board.SDA | — | PCA/IMU/ToF |
| ToF XSHUT | L/R/F/B | D16 / D25 / D26 / D24 | OUT | Addr 0x30/31/32/33 |

## Boot & Headless Start

Boot sequence: RED LED on → sensors init → GREEN LED on (ready). Headless: press Start (GPIO20 goes LOW) to arm readings_event and then loop_event.

## Robot photos (test version)

# System Architecture

Code structure at a glance:

- imports & global configuration (speeds, thresholds, filtering, timing); hardware setup (GPIO/I²C, PCA9685, MPU6050, pin constants)

- sensor initialization (ToF with XSHUT addressing and ultrasonic fallbacks)

- data model & threading primitives (Events, Locks, deques for plotting/logging)

- RobotController class (low‑level actuation, PWM/servo handling with optional slew limiting, distance filtering, lane‑keeping corrections, turn‑decision helpers)

- sensor_reader thread (continuous filtered measurements into a shared dict)

- robot_loop FSM (IDLE → CRUISE → TURN_INIT → TURNING → POST_TURN → STOPPED, with yaw integration, narrow‑mode scaling, and safety guards)

- GUI subsystem (Tkinter plots, status, sliders, CSV export, preset save/load); start/stop handlers; utilities (CSV export, JSON overrides)

- The __main__ entry (GUI vs headless startup, thread lifecycles, cleanup)

The stack is organized into two real-time threads plus an optional GUI:

- sensor_reader: acquires ultrasonic/ToF ranges, applies median+EMA smoothing with spike rejection, and publishes under a Lock.

- robot_loop: integrates gyro Z to yaw, runs the finite state machine (FSM), computes steering/motor commands via PCA9685.

- GUI (optional): Tkinter with live plots, tuning sliders, CSV export, and start/stop controls.

Synchronization uses Events (readings_event, loop_event, sensor_tick) to gate progress without busy-waiting. A servo slew limiter (deg/s) may be enabled to reduce oscillation on straights.

## Code Structure

### Imports

#### External Libraries

Uses standard threading/time/collections plus: gpiozero.DistanceSensor for ultrasounds; Adafruit PCA9685 for PWM (servo & motor); adafruit_mpu6050 for IMU; adafruit_vl53l0x for ToF; Tkinter + matplotlib for GUI/plots; numpy for median/mean; RPi.GPIO for LEDs/button; digitalio + board for XSHUT and I²C lines. These define the runtime I/O model.

#### Internal Modules

The solution is a single Python program structured into clear sections (imports, configuration, hardware setup, RobotController class, sensor thread, FSM loop, GUI, utilities, main). No separate packages are required; this keeps deployment simple on the Pi.

### Variables

#### Global Constants

Tunable thresholds & timings (e.g., SPEED_* per FSM state; SOFT_MARGIN/MAX_CORRECTION; STOP_THRESHOLD; FRONT_TURN_TRIGGER; TURN_DECISION_THRESHOLD; TURN_TIMEOUT; TURN_LOCKOUT; TARGET_TURN_ANGLE and tolerance; narrow-mode thresholds; filtering alphas/jumps; loop/sensor delays). These drive behavior and can be adjusted via GUI sliders or JSON overrides.

#### Robot-Specific Variables

Servo geometry (SERVO_CENTER/MIN/MAX and pulse widths), PCA channels for servo/motor, ultrasonic pins, ToF XSHUT pins, LEDs and Start button. Also stores immutable base values and environment scaling factors (SPEED_ENV_FACTOR, DIST_ENV_FACTOR) used by helper getters (eff_*). JSON overrides are loaded by load_variables_from_json() early so all downstream logic sees updated values.

### H/W Setup

#### Hardware Configuration

GPIO set to BCM mode; I²C bus initialized. PCA9685 set to 50 Hz (servo-friendly). IMU (MPU6050) instantiated and a 500-sample gyro bias is computed at startup (robot must be still). ToF sensors are optionally addressed via XSHUT (0x30..0x33) and started in continuous mode; fallback to ultrasonic is automatic if ToF init fails.

#### Pin Assignments

Front US (TRIG 22 / ECHO 23), Left (27 / 17), Right (5 / 6). Servo on PCA ch 0, motor driver on ch 1/2. Start button on GPIO20 (pull-up, active LOW). LEDs: GREEN 19, RED 13. I²C SCL/SDA via board.SCL/board.SDA.

### Robot Classes

#### Sensor Handling

RobotController.filtered_distance() fuses raw ToF/US with a small rolling history (median), jump rejection (FILTER_JUMP[_TOF]) and EMA smoothing (FILTER_ALPHA[_TOF]). Each sensor type uses its own parameters. Distances are normalized to cm; sentinel 999 denotes invalid/unavailable.

#### Motor Control

rotate_motor() maps 0–100% duty to PCA9685 channels (FWD/REV). stop_motor() zeros both channels. set_servo() clamps to limits, applies optional slew limiting (SERVO_SLEW_DPS in deg/s) to limit command rate, converts angle to microsecond pulse and to PCA duty cycle. safe_straight_control() produces bounded lane-keeping corrections based on SOFT_MARGIN and MAX_CORRECTION.

#### State Management

RobotState enum defines IDLE, CRUISE, TURN_INIT, TURNING, POST_TURN, STOPPED. Events (readings_event, loop_event) gate progression; sensor_tick coordinates waits to keep loops responsive without busy-waiting. Narrow-mode derives from L+R with hysteresis to scale speeds/thresholds.

### Robot Loop

#### Main Control Loop

robot_loop() runs continuously after readings_event is set. It integrates yaw from gyro Z (with a simple low-pass) and appends telemetry to deques for plotting. FSM actions drive servo/motor per state with state_speed_value() (affected by narrow-mode scaling). Lap/turn counters update, and POST_LAP behavior enforces stop at MAX_LAPS.

#### Decision Making

Turn preconditions: front < FRONT_TURN_TRIGGER and (t − last_turn) ≥ TURN_LOCKOUT. Direction requires XOR(open_left, open_right), where open is None/999 or > TURN_DECISION_THRESHOLD. Optional LOCK_TURN_DIRECTION fixes the first chosen side. Turn termination uses |Δyaw − TARGET_TURN_ANGLE| ≤ tolerance, else timeout or max-angle as fail-safes. STOP_THRESHOLD forces immediate STOPPED with timed auto-retry.

#### Movement Logic

Before the first turn, servo stays centered to avoid premature biasing. After the first turn, gentle wall-following corrections are applied when a side distance falls below SOFT_MARGIN; each correction is time-limited (CORRECTION_DURATION) and can renew if still needed. POST_TURN holds straight briefly before resuming CRUISE.

### GUI

#### Tkinter Setup

launch_gui() builds a 3-panel matplotlib view (front, sides, yaw) with live annotations, start/stop buttons, status indicators (including a circular traffic-light style), and labels for turns/laps. Clean shutdown stops ToF continuous mode and calls GPIO.cleanup().

#### Sliders & Data Visualization

Grouped sliders expose key parameters (speeds, margins, triggers, timeouts). Changes take effect live and can be saved/loaded as JSON presets. Export CSV captures time series, state, counters, and a snapshot of slider values for later analysis.

### Main

#### Initialization

The __main__ guard prints mode and launches GUI or headless path. In headless mode, LEDs indicate readiness; a physical Start press arms readings_event, then loop_event. KeyboardInterrupt and window close handlers stop loops, halt sensors, and clean up GPIO.

#### Robot Execution

In GUI mode, sensor and loop threads are started on demand via the buttons. In headless mode, sensor_reader runs as a daemon and robot_loop runs in the foreground after Start. Both modes share exactly the same control logic, ensuring identical behavior during competition.

# Decision‑Making Architecture

The controller separates sensing, eligibility checks, and action commits. Each behavior is guarded by explicit thresholds, hysteresis, and lockouts so that small sensor fluctuations do not cause thrashing. All important decisions are explainable in logs and reproducible by fixed parameter sets (or slider presets).

## Perception Pipeline

- Distances are read as meters (gpiozero) or mm (ToF) and normalized to cm.

- Median over N_READINGS suppresses outliers; EMA (FILTER_ALPHA) smooths; values jumping beyond FILTER_JUMP are rejected.

- For corridor reasoning we derive: (a) front distance, (b) left & right distances, and (c) left+right sum for narrow-mode.

Example calibration anchor: with 1.0 m corridor width and a 10 cm robot, left+right is expected near 90 cm; this informs NARROW_SUM_THRESHOLD setting.

## Corridor Model & Narrow‑Mode Scaling

We treat the track as four straight corridors and four corners. Narrow‑mode activates when L+R < NARROW_SUM_THRESHOLD and deactivates above threshold+NARROW_HYSTERESIS. When active, SPEED_ENV_FACTOR and DIST_ENV_FACTOR can scale state speeds and distance thresholds to keep the robot stable (e.g., drive slower and begin corrections earlier). Hysteresis prevents chatter when moving near the boundary of activation.

## Turn Eligibility (Pre‑conditions)

A turn is considered only when (i) the front distance falls below FRONT_TURN_TRIGGER and (ii) the last turn ended ≥ TURN_LOCKOUT seconds ago. This prevents premature or back‑to‑back turns on zig‑zag geometries.

## Direction Selection (Symmetry‑Aware)

The side ‘open’ predicate is: distance is None/999 or distance > TURN_DECISION_THRESHOLD. We require XOR(open_left, open_right) so that we only commit when exactly one side is clearly open; if both are open or both closed we keep waiting in TURN_INIT at a reduced speed. Optionally, LOCK_TURN_DIRECTION fixes the first chosen direction (LEFT/RIGHT) for the session for consistent lap direction.

## Turn Commit & Steering Command

Upon selection, we: (a) record yaw_start and time, (b) set the servo to TURN_ANGLE_LEFT/RIGHT (hardware setpoints), and (c) switch to state TURNING with SPEED_TURN. If servo slew limiting is enabled, the command rate is constrained to SERVO_SLEW_DPS to avoid mechanical shock and to reduce IMU spikes.

## Termination Strategy (Multi‑guard)

Primary: gyro‑based angle tracking. We estimate Δyaw = yaw − yaw_start and stop when |Δyaw − TARGET_TURN_ANGLE| ≤ TURN_ANGLE_TOLERANCE. Fallback 1: timeout when (now − turn_start) > TURN_TIMEOUT to avoid deadlocks. Fallback 2: a hard cap when |Δyaw| ≥ MAX_TURN_ANGLE to contain extreme cases (e.g., slipping, missing side). After stopping we center the servo, optionally snap yaw to the exact target for continuity, and enter POST_TURN for a short straight stabilization.

Rationale: IMU bias accumulates slowly, but at corner time‑scales the bias is quasi‑constant; using relative yaw is robust to drift over a single corner.

## Straight‑Line Corrections

After the first turn (to establish lane context), wall‑following applies only gentle, time‑boxed corrections when min(left,right) < SOFT_MARGIN. The correction magnitude saturates at MAX_CORRECTION for CORRECTION_DURATION; if still near the wall, it can renew. This prevents over‑steering on straights while maintaining a comfortable distance from the inner walls.

## Obstacle Stop & Auto‑Retry

If front < STOP_THRESHOLD at any time, motors stop and the state becomes STOPPED with a retry deadline now+OBSTACLE_WAIT_TIME. On expiry the controller rechecks front; if clear (≥ STOP_THRESHOLD) the robot resumes CRUISE.

## Determinism, Hysteresis, and Lockouts

- Determinism: state transitions require explicit pre‑conditions and never depend on transient, ambiguous readings.

- Hysteresis: both narrow‑mode and angle tolerance avoid edge‑case chatter.

- Lockouts: TURN_LOCKOUT guarantees spacing between turns; servo slew bounds steering rate.

# Finite State Machine (FSM)

| State | Enter | Actions | Exit |
| --- | --- | --- | --- |
| IDLE | Readings on, loop off | Motors off; servo centered | Loop on → CRUISE |
| CRUISE | Normal driving | Gentle wall‑following after 1st turn | Stop → STOPPED; Turn pre‑conditions → TURN_INIT |
| TURN_INIT | Front < trigger & lockout OK | Slow; wait XOR(open side) | Direction chosen → TURNING; Front re‑safe → CRUISE |
| TURNING | Direction committed | Servo setpoint; track yaw; guards | Angle tolerance OR timeout OR max‑angle → POST_TURN |
| POST_TURN | Turn finished | Short straight stabilization | Timer elapsed → CRUISE |
| STOPPED | Obstacle/user/laps | Motors off; optional auto‑retry | Retry clear → CRUISE |

# Parameters & Profiles

| Category | Key Parameters (defaults) | Purpose |
| --- | --- | --- |
| Speeds | SPEED_CRUISE/TURN_INIT/TURN/POST_TURN = 25/17/20/20 | State‑specific duty |
| Driving | SOFT_MARGIN=30 cm; MAX_CORRECTION=7°; CORRECTION_DURATION=0.25 s | Wall‑following |
| Safety | STOP_THRESHOLD=20 cm; TURN_TIMEOUT=4.5 s; TURN_LOCKOUT=1.5 s | Guards |
| Turns | FRONT_TURN_TRIGGER=90 cm; TURN_DECISION_THRESHOLD=100 cm | Eligibility & direction |
| Yaw | TARGET_TURN_ANGLE=85°; TURN_ANGLE_TOLERANCE=5°; MAX_TURN_ANGLE=120° | Termination |
| Narrow | NARROW_SUM_THRESHOLD=60 cm; NARROW_HYSTERESIS=10 cm | Scaling & chatter‑free toggling |
| Filters | FILTER_ALPHA(_TOF)=1.0; FILTER_JUMP(_TOF)=999; N_READINGS=1 | Smoothing & spike reject |
| Timing | LOOP_DELAY=0.001 s; SENSOR_DELAY=0.005 s | Loop pacing |

## Preset Profiles

| Profile | Cruise | Turn | Target° | Tol° | Front Trigger | Lockout |
| --- | --- | --- | --- | --- | --- | --- |
| Safe | 12 | 12 | 84 | 6 | 100 cm | 2.0 s |
| Balanced (default) | 15 | 15 | 86 | 5 | 90 cm | 1.5 s |
| Aggressive | 18 | 16 | 88 | 4 | 80 cm | 1.2 s |

# Calibration & Ops

- Gyro: keep robot still for 500‑sample bias calibration at boot.

- Servo: verify SERVO_CENTER and endpoints to avoid binding.

- Ranges: validate 20/50/100 cm; adjust FILTER_ALPHA/JUMP if noisy.

- Front trigger: increase until turns start safely without false positives.

- Yaw target: mark square corners; adjust TARGET_TURN_ANGLE, then refine with tolerance.

## Operations Runbook

Power → READY LED or GUI → Start Readings → Start Loop (or headless Start). Observe plots; export CSV and save slider presets after runs.

# KPIs & Test Protocol

| KPI | Target | Measurement |
| --- | --- | --- |
| Laps completed | 3/3 | lap_count; CSV |
| Avg yaw error/turn | ≤ 5° | compare Δyaw vs target |
| False emergency stops | ≤ 1 per run | STOP without obstacle |
| Narrow‑mode stability | No chatter | L+R hysteresis behavior |
| Lap time variance | ≤ 10% | std. dev. across laps |

Protocol: straight‑line baseline → single‑corner tuning → full‑lap with varying corridor widths → obstacle stop/retry → 3‑lap verification.

# Compliance (Rules Snapshot)

| Rule | Implementation | Status |
| --- | --- | --- |
| 3 autonomous laps | lap_count; MAX_LAPS; POST_LAP_DURATION | Compliant |
| Single Start button; no wireless during run | Headless GPIO20 start; GUI optional before run | Compliant |
| Respect driving direction; no wall movement | LOCK_TURN_DIRECTION; no wall control pins | Compliant |
| Obstacle stop | STOP_THRESHOLD + retry window | Compliant |

# Risks & Mitigations

Chattering near thresholds → add hysteresis or filtering; under/over turns → adjust target°/tolerance and verify IMU bias; late turns → raise front trigger or reduce turn‑init speed; oscillations on straights → lower MAX_CORRECTION or enable servo slew; sensor dropouts → check power/ground and fall back to ultrasonics if ToF fails.

# Diagrams

## Pinout / Wiring (Conceptual)

## Driving Area — State Map (Abstract)

## Decision Flowchart (FSM + Safety + Narrow Mode)

# Appendix A - JSON Override Example

{
  "SPEED_CRUISE": 25, "SPEED_TURN_INIT": 17, "SPEED_TURN": 20, "SPEED_POST_TURN": 20,
  "FRONT_TURN_TRIGGER": 90, "TURN_DECISION_THRESHOLD": 100,
  "TURN_ANGLE_LEFT": 60, "TURN_ANGLE_RIGHT": 120,
  "TARGET_TURN_ANGLE": 85, "TURN_ANGLE_TOLERANCE": 5,
  "TURN_TIMEOUT": 4.5, "TURN_LOCKOUT": 1.5,
  "SOFT_MARGIN": 30, "MAX_CORRECTION": 7, "STOP_THRESHOLD": 20,
  "NARROW_SUM_THRESHOLD": 60, "NARROW_HYSTERESIS": 10, "SERVO_SLEW_DPS": 0
}

# Appendix B - Variables

Notes:

- Many thresholds are dynamically scaled when the robot enters a narrow corridor. Speeds are multiplied by NARROW_FACTOR_SPEED (via SPEED_ENV_FACTOR) and distance thresholds by NARROW_FACTOR_DIST (via DIST_ENV_FACTOR). The code uses helper functions eff_* to compute effective values.

- Units: distances in centimeters (cm), time in seconds (s), angles in degrees (°), speeds as 0–100% PWM.

| Variable | Scope & Detailed Explanation | Accepted values & meanings |
| --- | --- | --- |
| — Initialization & Operating Mode — |  |  |
| USE_GUI | Selects runtime mode. If 1, starts the Tkinter GUI with live plots, sliders, and control buttons. If 0, runs headless and waits for the physical START button (GPIO). | 0 = headless (competition), 1 = GUI (debug). |
| USE_TOF_SIDES | Chooses sensor type for side distances. 1 initializes VL53L0X ToF for left/right; 0 uses ultrasonic. | 0 = ultrasonic, 1 = ToF (VL53L0X). |
| USE_TOF_FRONT | Chooses sensor type for the front distance sensor. | 0 = ultrasonic, 1 = ToF (VL53L0X). |
| — Narrow-Corridor Behavior — |  |  |
| NARROW_SUM_THRESHOLD | If (left + right) < threshold → narrow mode ON. | cm; typical 40–120. |
| NARROW_HYSTERESIS | Exit narrow mode only when (left + right) > threshold + hysteresis. | cm; typical 5–30. |
| NARROW_FACTOR | Legacy/general factor (not directly applied in current code). | Float ≥ 0.0 (unused). |
| NARROW_FACTOR_SPEED | Multiplier applied to state speeds in narrow mode (via SPEED_ENV_FACTOR). | Float ≥ 0.0; 1.0 = no change. |
| NARROW_FACTOR_DIST | Multiplier applied to distance thresholds in narrow mode (via DIST_ENV_FACTOR). | Float ≥ 0.0; 1.0 = no change. |
| — Speeds (0–100%) — |  |  |
| SPEED_IDLE | Motor command in IDLE. | 0–100; usually 0. |
| SPEED_STOPPED | Motor command in STOPPED. | 0–100; usually 0. |
| SPEED_CRUISE | Baseline forward speed during CRUISE. | 0–100. |
| SPEED_TURN_INIT | Speed while waiting for open side to turn. | 0–100. |
| SPEED_TURN | Speed during TURNING. | 0–100. |
| SPEED_POST_TURN | Speed after a turn during POST_TURN. | 0–100. |
| — Driving & Safety — |  |  |
| SOFT_MARGIN | Distance from wall when gentle steering corrections begin (safe_straight_control). | cm; typical 10–60. |
| MAX_CORRECTION | Max steering correction magnitude from safe_straight_control. | degrees; typical 3–15°. |
| CORRECTION_DURATION | How long a correction is held once engaged. | seconds; typical 0.1–0.5s. |
| STOP_THRESHOLD | Immediate stop if front distance < STOP_THRESHOLD. | cm; typical 10–50. |
| OBSTACLE_WAIT_TIME | Time to wait while stopped before retrying after an obstacle stop. | seconds; typical 2–8s. |
| — Turn Management — |  |  |
| FRONT_TURN_TRIGGER | Entering TURN_INIT when front < threshold and lockout allows. | cm; typical 70–130. |
| TURN_DECISION_THRESHOLD | Side considered open if None/999 or > threshold. | cm; typical 60–150. |
| TURN_ANGLE_LEFT | Servo target to initiate left turn in TURN_INIT. | degrees; ~55–90° (servo scale). |
| TURN_ANGLE_RIGHT | Servo target to initiate right turn in TURN_INIT. | degrees; ~90–125° (servo scale). |
| FRONT_SAFE_DISTANCE | Front distance considered safe to end a turn (kept for reuse; not active). | cm; 120–200+. |
| SIDE_SAFE_DISTANCE | Side distance considered safe to end a turn (kept for reuse; not active). | cm; 20–60. |
| TARGET_DISTANCE | Desired distance from walls (for potential centering). | cm; typical ~30. |
| TURN_TIMEOUT | Maximum duration allowed for TURNING before force stop. | seconds; 2–6s. |
| TURN_LOCKOUT | Minimum interval between consecutive turns. | seconds; 0.5–3s. |
| POST_TURN_DURATION | Straight driving duration after a turn. | seconds; 0.2–1.5s. |
| LOCK_TURN_DIRECTION | Lock first valid turn direction and keep using it. | 0 = off, 1 = on. |
| TARGET_TURN_ANGLE | Desired yaw change; reaching within tolerance ends the turn (Condition C). | degrees; 80–95°. |
| TURN_ANGLE_TOLERANCE | Allowed deviation from target to stop the turn. | degrees; 3–10°. |
| SERVO_SLEW_DPS | Servo slew rate limiter to smooth steering changes. | degrees/s; 0 = unlimited, ~150–450 typical. |
| MIN_TURN_ANGLE | Minimum yaw change before permitting stop (not currently enforced). | degrees; 60–80°. |
| MAX_TURN_ANGLE | Maximum yaw change; exceeding forces stop (Condition D). | degrees; 100–130°. |
| — Laps — |  |  |
| MAX_LAPS | Max laps before final stop (every 4 turns = 1 lap). 0 = unlimited. | Integer ≥ 0; 0 = ∞. |
| POST_LAP_DURATION | After last lap, drive forward briefly before stopping. | seconds; 0.5–2s. |
| — Sensor Filtering & Ranges — |  |  |
| N_READINGS | Median window size for sensor samples. | Integer ≥ 1; 1–7 typical. |
| US_QUEUE_LEN | gpiozero.DistanceSensor queue length for ultrasonic. | Integer ≥ 1; default 5. |
| FILTER_ALPHA | EMA smoothing for ultrasonic after median. | 0–1; 0.1 smooth, 1.0 none. |
| FILTER_JUMP | Max allowed jump between ultrasonic readings; bigger jumps treated as spikes. | cm; 10–200+, 999 disables. |
| FILTER_ALPHA_TOF | EMA smoothing for ToF after median. | 0–1; same as above. |
| FILTER_JUMP_TOF | Max allowed jump between ToF readings before spike rejection. | cm; 10–200+, 999 disables. |
| US_MAX_DISTANCE_FRONT | Ultrasonic max distance (meters) for front sensor (library parameter). | meters; ~2–4m. |
| US_MAX_DISTANCE_SIDE | Ultrasonic max distance (meters) for side sensors. | meters; ~1–3m. |
| — Loop & Plotting — |  |  |
| LOOP_DELAY | Main FSM loop delay; smaller = more responsive but more CPU. | seconds; 0.001–0.02. |
| SENSOR_DELAY | Sensor thread pacing; also used with sensor_tick. | seconds; 0.003–0.02. |
| MAX_POINTS | Max points stored/plotted for time series. | Integer; 300–1000 typical. |
| RAD2DEG | Constant for rad→deg conversion during yaw integration. | ≈57.2958 (fixed). |
| — Config & Scaling Bases — |  |  |
| CONFIG_FILE | External JSON used to override defaults at startup. Only keys present in globals() are applied. | Path string; default BASE_DIR/1st_mission_variables.json. |
| BASE_SPEED_* | Frozen copies of speed variables captured after JSON load; used as bases for scaling. | Derived from SPEED_* at load time. |
| BASE_MAX_CORRECTION / BASE_FRONT_TURN_TRIGGER / BASE_STOP_THRESHOLD | Frozen base values for scaling distance thresholds and corrections. | Derived at load time. |
| SPEED_ENV_FACTOR | Global multiplier for speeds; set automatically by corridor detector. | Float ≥ 0.0; 1.0 normal, NARROW_FACTOR_SPEED in narrow mode. |
| DIST_ENV_FACTOR | Global multiplier for distance thresholds; set automatically by corridor detector. | Float ≥ 0.0; 1.0 normal, NARROW_FACTOR_DIST in narrow mode. |
| — Hardware & I/O Constants — |  |  |
| START_BTN | BCM GPIO pin for physical start button (headless mode). | BCM number; default 20. |
| GREEN_LED / RED_LED | BCM GPIO pins for status LEDs. | BCM numbers; GREEN=19, RED=13. |
| TRIG_FRONT/ECHO_FRONT; TRIG_LEFT/ECHO_LEFT; TRIG_RIGHT/ECHO_RIGHT | BCM GPIO pin pairs for ultrasonic trigger/echo. | FRONT (22,23), LEFT (27,17), RIGHT (5,6). |
| MOTOR_FWD / MOTOR_REV | PCA9685 channels for motor forward and reverse PWM. | 0–15; default FWD=1, REV=2. |
| SERVO_CHANNEL | PCA9685 channel for steering servo PWM. | 0–15; default 0. |
| SERVO_CENTER / SERVO_MIN_ANGLE / SERVO_MAX_ANGLE | Servo trim/limits used for clamping and centering. | Degrees; CENTER≈88, MIN≈50, MAX≈130 (tune). |
| SERVO_PULSE_MIN/MAX / SERVO_PERIOD | PWM μs timings mapped to min/max angles over period. | μs; 1000/2000 over 20000 (50Hz). |
| — Runtime Control Flags & Counters — |  |  |
| readings_event / loop_event / sensor_tick | Threading Events controlling acquisition, FSM permission, and pacing/wakeup. | Boolean events; set()/clear() by GUI/code. |
| status_text | Human-readable status displayed in GUI. | String; internal. |
| turn_count / lap_count | Counters for turns and laps (4 turns = 1 lap). | Integers ≥ 0; internal. |
| stop_reason | Reason for stop affecting retry logic. | String: 'USER', 'OBSTACLE', or 'LAPS'. |
| obstacle_wait_deadline | Monotonic time after which to retry when stopped by obstacle. | Float seconds; internal. |

# Appendix C - Source Code Analysis

1) Imports & global configuration

- External libs: RPi.GPIO, gpiozero.DistanceSensor (ultrasonic, meters), adafruit_pca9685 (PWM @ 50 Hz), adafruit_mpu6050 (gyro Z in rad/s), adafruit_vl53l0x (ToF in mm), board/digitalio (I²C + XSHUT), tkinter + matplotlib (GUI/plots), numpy (median/mean), threading, collections.deque, csv, json, time.

- Configuration constants: Speed setpoints per FSM state (SPEED_*), turn thresholds (FRONT_TURN_TRIGGER, TURN_DECISION_THRESHOLD), safety guards (STOP_THRESHOLD, TURN_TIMEOUT, TURN_LOCKOUT, MAX_TURN_ANGLE), yaw targets (TARGET_TURN_ANGLE, TURN_ANGLE_TOLERANCE), narrow-mode thresholds (NARROW_SUM_THRESHOLD, NARROW_HYSTERESIS), filtering (FILTER_ALPHA(_TOF), FILTER_JUMP(_TOF), N_READINGS), timing (LOOP_DELAY, SENSOR_DELAY), servo geometry and pulses (SERVO_*, 1000–2000 µs).

- Overrides: load_variables_from_json() runs immediately after defaults so all subsequent logic uses injected values. Only keys already present in globals() are applied.

2) Hardware setup (GPIO/I²C, pin map)

- GPIO mode: BCM; LEDs on GPIO19(green), GPIO13(red); start button GPIO20 (pull-up, active-LOW).

- I²C devices: PCA9685 (pca.frequency = 50), MPU6050, optional VL53L0X sensors.

- Ultrasonic pins: front (TRIG22,ECHO23), left (27,17), right (5,6) via gpiozero.DistanceSensor (distance in meters, max_distance set per front/side).

- PWM channels: servo on PCA ch 0; motor FWD/REV on PCA ch 1/2.

- IMU bias: 500 samples averaged at boot to compute Z-gyro bias.

3) Sensor initialization (ToF w/ XSHUT, ultrasonic fallback)

- ToF addressing: per-sensor XSHUT pins (D16/D25/D26/D24) power-cycle and set unique I²C addresses 0x30–0x33, then start_continuous() with a 20 ms budget.

- Fallback: any ToF init exception disables ToF flags and uses ultrasonics for the affected directions.

- Mixed mode: front/side ToF enablement is independent; you can run US on sides and ToF on front (or vice-versa).

4) Data model & threading primitives

- Shared sensor snapshot: sensor_data = {"front":…, "left":…, "right":…} guarded by a Lock.

- Events: readings_event (sensors armed), loop_event (FSM armed), sensor_tick (timed wait with immediate wake for quick responsiveness).

- Telemetry: deques (time_data, front/left/right_data, angle_data, state_data) with MAX_POINTS cap for plots/CSV.

5) RobotController (low-level actuation & signal processing)

- Servo command: set_servo(angle) clamps to [SERVO_MIN_ANGLE, SERVO_MAX_ANGLE]; optional slew limiting:
angle_step ≤ SERVO_SLEW_DPS * dt, where dt is computed via monotonic_ns().
Pulse conversion:
pulse_us = SERVO_PULSE_MIN + (SERVO_PULSE_MAX - SERVO_PULSE_MIN) * (angle - MIN)/(MAX - MIN) → PCA duty (0..65535).

- Motor command: rotate_motor(speed) maps 0–100% to PCA duty; selects FWD vs REV channel by sign; stop_motor() zeros both.

- Distance filtering: filtered_distance(sensor, history, attr, sensor_type):

  - Read (us: distance * 100 cm; tof: range/10 cm); append to bounded history.

  - Median over last N_READINGS; reject spikes >|FILTER_JUMP| vs previous; EMA: y = α·x + (1−α)·y_prev.

  - Returns smoothed cm or 999 sentinel if no valid data.

- Lane-keeping: safe_straight_control(l,r) computes bounded correction using eff_soft_margin() and eff_max_correction() (scaled by narrow-mode factors), returns a target servo angle.

- Turn helper: turn_decision(l,r) → "LEFT", "RIGHT", or None using XOR(open_left, open_right) where “open” is (None or 999 or > TURN_DECISION_THRESHOLD).

6) Real-time threads

- sensor_reader (period ≈ SENSOR_DELAY):

  - Acquire left/right/front from ToF or US, run the filter, write to sensor_data under Lock.

  - Sleeps with sensor_tick.wait(SENSOR_DELAY) (can be preemptively woken).

- robot_loop (soft-real-time main loop):

  - If readings_event not set → hold IDLE.

  - On each tick: copy snapshot, update yaw, narrow-mode, telemetry, then run FSM if loop_event is set.

7) Yaw integration & units

- IMU input: gyro_z in rad/s; bias-corrected, low-passed (ALPHA); integrated to degrees:
yaw += (gyro_z_filtered * RAD2DEG) * dt, where RAD2DEG = 180/π, dt from monotonic_ns().

8) Finite State Machine (FSM)

- States: IDLE → CRUISE → TURN_INIT → TURNING → POST_TURN → STOPPED.

- Key transitions & guards:

  - Emergency stop (anywhere): front < eff_stop_threshold() → STOPPED, schedule auto-retry for OBSTACLE_WAIT_TIME.

  - Turn eligibility (CRUISE → TURN_INIT): front < eff_front_turn_trigger() and (now − last_turn_time) ≥ TURN_LOCKOUT.

  - Direction selection (TURN_INIT): require XOR(open_left, open_right); if LOCK_TURN_DIRECTION=1, fix first chosen direction for the run; else keep waiting at SPEED_TURN_INIT.

  - Turn commit (→ TURNING): set servo to TURN_ANGLE_LEFT/RIGHT, record turn_start_yaw/time, run at SPEED_TURN.

  - Turn termination (TURNING → POST_TURN):

    - Primary: |Δyaw − TARGET_TURN_ANGLE| ≤ TURN_ANGLE_TOLERANCE

    - Failsafes: (now − turn_start_time) > TURN_TIMEOUT or |Δyaw| ≥ MAX_TURN_ANGLE

    - On stop: center servo, last_turn_time = now, optionally snap yaw to exact target for continuity, increment turn_count; every 4 turns → lap_count++ (stop after MAX_LAPS with POST_LAP_DURATION forward).

  - Post-turn (POST_TURN → CRUISE): hold straight for POST_TURN_DURATION.

9) Environment adaptation (narrow-mode)

- Detector: compute sum_lr = left + right for valid readings; enter if < NARROW_SUM_THRESHOLD, exit if > threshold + NARROW_HYSTERESIS.

- Scaling: set SPEED_ENV_FACTOR and DIST_ENV_FACTOR; all eff_* helpers (e.g., eff_soft_margin(), eff_front_turn_trigger()) and state_speed_value() consume these to adapt behavior without changing base constants.

10) GUI subsystem (for debugging)

- Tkinter UI: buttons (Start Readings, Start Loop, Stop Loop), status label, circular state indicator, turn/lap counters.

- Plots: 3 Matplotlib subplots (front, side [left vs −right], yaw); dynamic x-window, last-value annotations, and “after()” updates (100–200 ms).

- Sliders: grouped controls for speeds, margins, triggers, timeouts; write through to globals(); save/load JSON presets.

- Export: export_data_csv() writes time series + current state + slider snapshot for reproducibility.

- Shutdown: on_closing() calls stop_loop(), stops ToF continuous mode, GPIO.cleanup().

11) Start/stop & lifecycle

- Handlers: start_readings() spawns sensor_reader (daemon) and robot_loop (daemon) once; start_loop() sets loop_event; stop_loop() clears loop, records stop_reason, resets counters, unlocks direction, centers servo, stops motor.

- Headless path (__main__): LEDs signal readiness; wait for physical Start (active-LOW); then set events and run robot_loop() in the foreground. KeyboardInterrupt and GUI close lead to clean shutdown.

12) Synchronization & timing

- Progress gating: readings_event must be set before any motion; loop_event controls whether FSM drives motors (plots still update while off).

- Wait strategy: sensor_tick.wait(…) used instead of time.sleep() so stop_loop() can set() to break out immediately.

- Typical periods: SENSOR_DELAY ≈ 5 ms (up to ~200 Hz raw read cadence); main loop LOOP_DELAY ≈ 1 ms cap (effective rate bounded by processing and GUI).

13) Safety/fallbacks

- Bounds: all servo angles clamped; motor duty bounded to 0–100%.

- Guards: emergency stop, turn timeout, max turn angle, turn lockout; narrow-mode scaling to reduce risk in tight corridors.

- Sensor resilience: jump-rejection, EMA smoothing, 999 sentinel for invalid; automatic ToF→US fallback on init failure.

- Thread summary

- sensor_reader — acquisition + filtering (median → spike reject → EMA) → publish under Lock.

- robot_loop — yaw integration (rad/s → deg), narrow-mode, FSM, PWM outputs via PCA9685, telemetry append.

- GUI (optional) — plotting, parameter sliders with live writes, CSV export, start/stop controls.

Events: readings_event, loop_event, sensor_tick avoid busy-waiting and keep latency low.
Servo slew (SERVO_SLEW_DPS) is optional; when enabled it caps ∆angle/∆t to damp oscillations and reduce IMU shock during aggressive steering.

# Appendix D - Source code

0001 | """

0002 | -----------------------------------------------------------------------------------------------------------------------

0003 | Autonomous drive 1st Mission WRO 2025 FE - VivaLaVida

0004 | v5.7

0005 | -----------------------------------------------------------------------------------------------------------------------

0006 | """

0007 |

0008 | # ===============================

0009 | # IMPORTS

0010 | # ===============================

0011 |

0012 | import threading

0013 | import time

0014 | import tkinter as tk

0015 | from collections import deque

0016 | import RPi.GPIO as GPIO

0017 | from board import SCL, SDA

0018 | import busio

0019 | from adafruit_pca9685 import PCA9685

0020 | import adafruit_mpu6050

0021 | import matplotlib.pyplot as plt

0022 | from matplotlib.backends.backend_tkagg import FigureCanvasTkAgg

0023 | import csv

0024 | from datetime import datetime

0025 | import json

0026 | import os

0027 | import tkinter.filedialog as fd

0028 | import board

0029 | import digitalio

0030 | import adafruit_vl53l0x

0031 | from enum import Enum, auto

0032 | import numpy as np

0033 | from gpiozero import DistanceSensor

0034 | #from gpiozero import Device

0035 | #from gpiozero.pins.pigpio import PiGPIOFactory

0036 | from threading import Event

0037 |

0038 | BASE_DIR = os.path.dirname(os.path.abspath(__file__)) # Get directory of the running script

0039 |

0040 | # ===============================

0041 | # CONFIGURATION VARIABLES

0042 | # ===============================

0043 |

0044 | # ---------- Initialization ----------

0045 | USE_GUI = 1                   # 1 = Debugging mode (run with GUI), 0 = COMPETITION MODE (run headless, no GUI)

0046 | USE_TOF_SIDES = 0             # Side sensors:  0 = Ultrasonic, 1 = ToF

0047 | USE_TOF_FRONT = 0             # Front sensor:  0 = Ultrasonic, 1 = ToF

0048 |

0049 | NARROW_SUM_THRESHOLD = 60     # cm; left+right distance threshold to decide if in between narrow walls

0050 | NARROW_HYSTERESIS = 10        # cm; prevents rapid toggling

0051 | NARROW_FACTOR = 1.0           # multiply by this when narrow, eg speed * factor

0052 | NARROW_FACTOR_SPEED = 1.0     # multiply state speeds when in narrow corridors

0053 | NARROW_FACTOR_DIST  = 1.0     # multiply distance-based thresholds/corrections when in narrow corridors

0054 |

0055 | # ---------- Speeds ----------

0056 | SPEED_IDLE = 0

0057 | SPEED_STOPPED = 0

0058 | SPEED_CRUISE = 25            # Motor speed for normal straight driving (0-100%)

0059 | SPEED_TURN_INIT = 17          # Motor speed while waiting for open side to turn

0060 | SPEED_TURN = 20               # Motor speed while turning

0061 | SPEED_POST_TURN = 20          # Motor speed following a turn

0062 |

0063 | # ---------- Driving ----------

0064 | SOFT_MARGIN = 30              # Distance from wall where small steering corrections start (cm)

0065 | MAX_CORRECTION = 7            # Maximum servo correction applied for wall-following (degrees)

0066 | CORRECTION_DURATION = 0.25    # How long a side-correction is held (seconds)

0067 | STOP_THRESHOLD = 20           # Front distance (cm) at which robot stops immediately

0068 | OBSTACLE_WAIT_TIME = 5.0      # seconds to wait before retrying after a front-stop

0069 |

0070 | # ---------- Turn management ----------

0071 | FRONT_TURN_TRIGGER = 90       # Front distance (cm) at which a turn is triggered

0072 | TURN_DECISION_THRESHOLD = 100  # Minimum side distance (cm) to allow turn in that direction

0073 | TURN_ANGLE_LEFT = 60          # Servo angle for left turn

0074 | TURN_ANGLE_RIGHT = 120        # Servo angle for right turn

0075 | FRONT_SAFE_DISTANCE = 160     # Front distance considered safe to end a turn (cm)

0076 | SIDE_SAFE_DISTANCE = 30       # Side distance considered safe to end a turn (cm)

0077 | TARGET_DISTANCE = 30          # Desired distance from side walls (cm)

0078 | TURN_TIMEOUT = 4.5            # Maximum time allowed for a turn (seconds)

0079 | TURN_LOCKOUT = 1.5            # Minimum interval between consecutive turns (seconds)

0080 | POST_TURN_DURATION = 0.5      # Time to drive straight after a turn (seconds)

0081 | LOCK_TURN_DIRECTION = 1       # 1 = enable turn lock direction after 1st turn, 0 = disable

0082 | TARGET_TURN_ANGLE = 85        # Degrees to turn per corner

0083 | TURN_ANGLE_TOLERANCE = 5      # Acceptable overshoot (degrees)

0084 |

0085 | SERVO_SLEW_DPS = 0        # max degrees/second the servo command may change (0 = no limit)

0086 | # Reduce to ~200 for gentler turns, increase to ~450 for sharper response.

0087 | # Set to 0 to disable limiting and go back to “as fast as the servo can”.

0088 |

0089 | # ---------- Turn angle constraints ----------

0090 | MIN_TURN_ANGLE = 65           # Minimum yaw change (degrees) before turn can stop

0091 | MAX_TURN_ANGLE = 120          # Maximum yaw change (degrees) to force stop turn

0092 |

0093 | # ---------- Laps ----------

0094 | MAX_LAPS = 3                  # Maximum number of laps before stopping (0 = unlimited)

0095 | POST_LAP_DURATION = 1       # Time to drive forward after final lap before stopping (seconds)

0096 |

0097 | # ---------- Sensor data filtering ----------

0098 | N_READINGS = 1                # Number of readings stored for median filtering

0099 | US_QUEUE_LEN = 5              # Queue_len for Ultrasonic gpiozero.DistanceSensor

0100 | FILTER_ALPHA = 1.0            # Ultrasonic 0.1 = smoother, 0.5 = faster reaction, 1 to ignore filter

0101 | FILTER_JUMP = 999             # Ultrasonic maximum jump (cm) allowed between readings

0102 | FILTER_ALPHA_TOF = 1.0        # ToF 0.1 = smoother, 0.5 = faster reaction, 1 to ignore filter

0103 | FILTER_JUMP_TOF = 999         # ToF maximum jump (cm) allowed between readings

0104 | US_MAX_DISTANCE_FRONT = 3.43  # Ultrasonic front max read distance

0105 | US_MAX_DISTANCE_SIDE = 1.72   # Ultrasonic side max read distance

0106 |

0107 | # ---------- Loop timing ----------

0108 | LOOP_DELAY = 0.001            # Delay between main loop iterations (seconds) 0.02

0109 | SENSOR_DELAY = 0.005          # Delay between sensor reads

0110 |

0111 | #-----------------------------------------------------------------------------------------

0112 |

0113 | # ---------- Plotting ----------

0114 | MAX_POINTS = 500               # Maximum number of points to store for plotting

0115 |

0116 | RAD2DEG = 180.0 / 3.141592653589793

0117 |

0118 | # ===============================

0119 | # LOAD VARIABLES FROM JSON (if exists)

0120 | # ===============================

0121 |

0122 | CONFIG_FILE = os.path.join(BASE_DIR, "1st_mission_variables.json")

0123 |

0124 | def load_variables_from_json(path=CONFIG_FILE):

0125 |     """

0126 |     Override defaults with values from JSON if the file exists.

0127 |     If missing or invalid, keep the built-in defaults from the code.

0128 |     """

0129 |     if not os.path.exists(path):

0130 |         print("⚙️ No external config found — using built-in defaults.")

0131 |         return

0132 |     try:

0133 |          with open(path, "r") as f:

0134 |               data = json.load(f)

0135 |          count = 0

0136 |          for k, v in data.items():

0137 |              if k in globals():

0138 |                  globals()[k] = v

0139 |                  count += 1

0140 |          print(f"✅ Loaded {count} variables from {os.path.basename(path)}")

0141 |      except Exception as e:

0142 |          print(f"[ERROR] Config load failed: {e} — using built-in defaults.")

0143 |

0144 | # Call it right after defining defaults so everything below sees the overrides

0145 | load_variables_from_json()

0146 |

0147 | # --- Store immutable base values for scaling ---

0148 | BASE_SPEED_IDLE        = SPEED_IDLE

0149 | BASE_SPEED_STOPPED     = SPEED_STOPPED

0150 | BASE_SPEED_CRUISE      = SPEED_CRUISE

0151 | BASE_SPEED_TURN_INIT   = SPEED_TURN_INIT

0152 | BASE_SPEED_TURN        = SPEED_TURN

0153 | BASE_SPEED_POST_TURN   = SPEED_POST_TURN

0154 |

0155 | BASE_MAX_CORRECTION     = MAX_CORRECTION

0156 | BASE_FRONT_TURN_TRIGGER = FRONT_TURN_TRIGGER

0157 | BASE_STOP_THRESHOLD     = STOP_THRESHOLD

0158 |

0159 | # --- Global factors (automatically updated) ---

0160 | SPEED_ENV_FACTOR = 1.0

0161 | DIST_ENV_FACTOR  = 1.0  # for distance-related thresholds

0162 |

0163 | # --- Helper functions to get "effective" (scaled) values ---

0164 | def eff_soft_margin():

0165 |     """Return scaled soft margin distance (cm) for gentle steering corrections."""

0166 |     return int(SOFT_MARGIN * DIST_ENV_FACTOR)

0167 |

0168 | def eff_max_correction():

0169 |     """Return scaled maximum steering correction (degrees)."""

0170 |     return max(1, int(MAX_CORRECTION * DIST_ENV_FACTOR))

0171 |

0172 | def eff_front_turn_trigger():

0173 |     """Return scaled front distance threshold (cm) for initiating a turn."""

0174 |     return int(FRONT_TURN_TRIGGER * DIST_ENV_FACTOR)

0175 |

0176 | def eff_stop_threshold():

0177 |     """Return scaled front distance threshold (cm) for immediate stop."""

0178 |     return int(STOP_THRESHOLD * DIST_ENV_FACTOR)

0179 |

0180 | # --- Scaled state speeds ---

0181 | def state_speed_value(state_name: str) -> int:

0182 |     base = {

0183 |         "IDLE":      SPEED_IDLE,

0184 |         "STOPPED":   SPEED_STOPPED,

0185 |         "CRUISE":    SPEED_CRUISE,

0186 |         "TURN_INIT": SPEED_TURN_INIT,

0187 |         "TURNING":   SPEED_TURN,

0188 |         "POST_TURN": SPEED_POST_TURN,

0189 |     }.get(state_name, SPEED_CRUISE)

0190 |     return int(base * SPEED_ENV_FACTOR)

0191 |

0192 | # ===============================

0193 | # HARDWARE SETUP

0194 | # ===============================

0195 |

0196 | GPIO.setmode(GPIO.BCM)

0197 | GPIO.setwarnings(False)

0198 | i2c = busio.I2C(board.SCL, board.SDA)

0199 |

0200 | START_BTN = 20                    #Button No1

0201 | GPIO.setup(START_BTN, GPIO.IN, pull_up_down=GPIO.PUD_UP)

0202 |

0203 | # ---------- LEDs ----------

0204 | GREEN_LED = 19

0205 | RED_LED   = 13

0206 | GPIO.setup(GREEN_LED, GPIO.OUT, initial=GPIO.LOW)

0207 | GPIO.setup(RED_LED,   GPIO.OUT, initial=GPIO.LOW)

0208 |

0209 | # ---------- Ultrasonic sensor pins ----------

0210 | TRIG_FRONT, ECHO_FRONT = 22, 23  # GPIO pins for front sensor

0211 | TRIG_LEFT, ECHO_LEFT = 27, 17    # GPIO pins for left sensor

0212 | TRIG_RIGHT, ECHO_RIGHT = 5, 6    # GPIO pins for right sensor

0213 |

0214 | # ---------- Motor ----------

0215 | MOTOR_FWD = 1                 # PCA9685 channel for forward motor control

0216 | MOTOR_REV = 2                 # PCA9685 channel for reverse motor control

0217 |

0218 | # ---------- Servo ----------

0219 | SERVO_CHANNEL = 0             # PCA9685 channel controlling steering servo

0220 | SERVO_CENTER = 88             # Neutral servo angle (degrees)

0221 | SERVO_MIN_ANGLE = 50          # Minimum physical servo angle (degrees)

0222 | SERVO_MAX_ANGLE = 130         # Maximum physical servo angle (degrees)

0223 | SERVO_PULSE_MIN = 1000        # Minimum PWM pulse width (microseconds)

0224 | SERVO_PULSE_MAX = 2000        # Maximum PWM pulse width (microseconds)

0225 | SERVO_PERIOD = 20000          # Servo PWM period (microseconds)

0226 |

0227 | # ===============================

0228 | # SENSOR INITIALIZATION (ToF + Ultrasonic, flexible for mixed scenarios)

0229 | # ===============================

0230 |

0231 | vl53_left = vl53_right = vl53_front = vl53_back = None

0232 | us_left = us_right = us_front = None

0233 |

0234 | try:

0235 |     # --------- ToF sensors ----------

0236 |     if USE_TOF_SIDES or USE_TOF_FRONT:

0237 |         print("Initializing VL53L0X ToF sensors...")

0238 |         # XSHUT pins setup (for powering sensors individually)

0239 |         xshut_left  = digitalio.DigitalInOut(board.D16)

0240 |         xshut_right = digitalio.DigitalInOut(board.D25)

0241 |         xshut_front = digitalio.DigitalInOut(board.D26)

0242 |         xshut_back  = digitalio.DigitalInOut(board.D24)

0243 |         for xshut in [xshut_left, xshut_right, xshut_front, xshut_back]:

0244 |             xshut.direction = digitalio.Direction.OUTPUT

0245 |             xshut.value = False  # power down

0246 |         time.sleep(0.1)

0247 |

0248 |         # Initialize left ToF if needed

0249 |         if USE_TOF_SIDES:

0250 |             xshut_left.value = True

0251 |             time.sleep(0.05)

0252 |             vl53_left = adafruit_vl53l0x.VL53L0X(i2c)

0253 |             vl53_left.set_address(0x30)

0254 |             vl53_left.measurement_timing_budget = 20000

0255 |             vl53_left.start_continuous()

0256 |             print("✅ Left ToF sensor set to address 0x30")

0257 |

0258 |         # Initialize right ToF if needed

0259 |         if USE_TOF_SIDES:

0260 |             xshut_right.value = True

0261 |             time.sleep(0.05)

0262 |             vl53_right = adafruit_vl53l0x.VL53L0X(i2c)

0263 |             vl53_right.set_address(0x31)

0264 |             vl53_right.measurement_timing_budget = 20000

0265 |             vl53_right.start_continuous()

0266 |             print("✅ Right ToF sensor set to address 0x31")

0267 |

0268 |         # Initialize front ToF if needed

0269 |         if USE_TOF_FRONT:

0270 |             xshut_front.value = True

0271 |             time.sleep(0.05)

0272 |             vl53_front = adafruit_vl53l0x.VL53L0X(i2c)

0273 |             vl53_front.set_address(0x32)

0274 |             vl53_front.measurement_timing_budget = 20000

0275 |             vl53_front.start_continuous()

0276 |             print("✅ Front ToF sensor set to address 0x32")

0277 |

0278 |         # Initialize back ToF (optional, if you want to use it)

0279 |         xshut_back.value = True

0280 |         time.sleep(0.05)

0281 |         vl53_back = adafruit_vl53l0x.VL53L0X(i2c)

0282 |         vl53_back.set_address(0x33)

0283 |         vl53_back.measurement_timing_budget = 20000

0284 |         vl53_back.start_continuous()

0285 |         print("✅ Back ToF sensor set to address 0x33")

0286 |

0287 | except Exception as e:

0288 |     print(f"[ERROR] VL53L0X initialization failed: {e}")

0289 |     # fallback to ultrasonic if ToF fails

0290 |     USE_TOF_SIDES = 0

0291 |     USE_TOF_FRONT = 0

0292 |     vl53_left = vl53_right = vl53_front = vl53_back = None

0293 |

0294 | time.sleep(0.2)

0295 |

0296 | # --------- Ultrasonic sensors ----------

0297 | # Initialize ultrasonic sensors only if that sensor is set to ultrasonic

0298 | if not USE_TOF_FRONT:

0299 |     us_front = DistanceSensor(echo=ECHO_FRONT, trigger=TRIG_FRONT, max_distance=US_MAX_DISTANCE_FRONT, queue_len=US_QUEUE_LEN)

0300 | if not USE_TOF_SIDES:

0301 |     us_left  = DistanceSensor(echo=ECHO_LEFT,  trigger=TRIG_LEFT,  max_distance=US_MAX_DISTANCE_SIDE, queue_len=US_QUEUE_LEN)

0302 |     us_right = DistanceSensor(echo=ECHO_RIGHT, trigger=TRIG_RIGHT, max_distance=US_MAX_DISTANCE_SIDE, queue_len=US_QUEUE_LEN)

0303 |

0304 | print("Sensors initialized:")

0305 | print(f"  Front: {'ToF' if USE_TOF_FRONT else 'Ultrasonic'}")

0306 | print(f"  Left : {'ToF' if USE_TOF_SIDES else 'Ultrasonic'}")

0307 | print(f"  Right: {'ToF' if USE_TOF_SIDES else 'Ultrasonic'}")

0308 |

0309 | pca = PCA9685(i2c)

0310 | pca.frequency = 50

0311 | mpu = adafruit_mpu6050.MPU6050(i2c)

0312 |

0313 | # ---------- Calibrate gyro bias at startup ----------

0314 | print("Calibrating gyro...")

0315 | N = 500

0316 | bias = 0

0317 | for _ in range(N):

0318 |     bias += mpu.gyro[2]

0319 |     time.sleep(0.005)

0320 | bias /= N

0321 | #print(f"Gyro bias: {bias}")

0322 |

0323 | # ===============================

0324 | # CONTROL FLAGS & STORAGE

0325 | # ===============================

0326 |

0327 | STATE_SPEED = {

0328 |     "IDLE": SPEED_IDLE,

0329 |     "STOPPED": SPEED_STOPPED,

0330 |     "CRUISE": SPEED_CRUISE,

0331 |     "TURN_INIT": SPEED_TURN_INIT,

0332 |     "TURNING": SPEED_TURN,

0333 |     "POST_TURN": SPEED_POST_TURN

0334 | }

0335 |

0336 | # ---------- CONTROL FLAGS ----------

0337 | readings_event = Event()

0338 | loop_event = Event()

0339 | sensor_tick = Event()   # used to wake waits immediately

0340 | status_text = "Idle"

0341 | turn_count = 0

0342 | lap_count = 0

0343 | # Reason-aware stopping (USER / OBSTACLE / LAPS)

0344 | stop_reason = None

0345 | obstacle_wait_deadline = 0.0

0346 |

0347 | # ---------- SENSOR DATA STORAGE ----------

0348 | time_data = deque(maxlen=MAX_POINTS)

0349 | front_data = deque(maxlen=MAX_POINTS)

0350 | left_data = deque(maxlen=MAX_POINTS)

0351 | right_data = deque(maxlen=MAX_POINTS)

0352 | angle_data = deque(maxlen=MAX_POINTS)

0353 | state_data = deque(maxlen=MAX_POINTS)

0354 |

0355 | # ===============================

0356 | # THREADED SENSOR READING

0357 | # ===============================

0358 | sensor_data = {

0359 |     "front": None,

0360 |     "left": None,

0361 |     "right": None

0362 | }

0363 | sensor_lock = threading.Lock()

0364 |

0365 | # ===============================

0366 | # ROBOT CONTROLLER

0367 | # ===============================

0368 |

0369 | class RobotController:

0370 |     def __init__(self, pca):

0371 |         self.pca = pca

0372 |         self.front_history = deque(maxlen=N_READINGS)

0373 |         self.left_history = deque(maxlen=N_READINGS)

0374 |         self.right_history = deque(maxlen=N_READINGS)

0375 |         self.smooth_left = None

0376 |         self.smooth_right = None

0377 |         self.smooth_front = None

0378 |         self.d_front = None

0379 |         self.d_left = None

0380 |         self.d_right = None

0381 |         self.sensor_index = 0

0382 |         self.gyro_z_prev = 0

0383 |         self._servo_last_angle = SERVO_CENTER

0384 |         self._servo_last_ns = time.monotonic_ns()

0385 |

0386 |     def set_filter_size(self, n):

0387 |         self.front_history = deque(list(self.front_history)[-n:], maxlen=n)

0388 |         self.left_history = deque(list(self.left_history)[-n:], maxlen=n)

0389 |         self.right_history = deque(list(self.right_history)[-n:], maxlen=n)

0390 |

0391 |     #def set_servo(self, angle):

0392 |     #    angle = max(SERVO_MIN_ANGLE, min(SERVO_MAX_ANGLE, angle))

0393 |     #    pulse = int(SERVO_PULSE_MIN + (SERVO_PULSE_MAX - SERVO_PULSE_MIN) *

0394 |     #                ((angle - SERVO_MIN_ANGLE) / (SERVO_MAX_ANGLE - SERVO_MIN_ANGLE)))

0395 |     #    self.pca.channels[SERVO_CHANNEL].duty_cycle = int(pulse * 65535 / SERVO_PERIOD)

0396 |

0397 |     def set_servo(self, angle):

0398 |         # 1) clamp target

0399 |         target = max(SERVO_MIN_ANGLE, min(SERVO_MAX_ANGLE, angle))

0400 |

0401 |         # 2) simple built-in slew limiter using one knob: SERVO_SLEW_DPS

0402 |         now_ns = time.monotonic_ns()

0403 |         dt = max(0.0, (now_ns - self._servo_last_ns) * 1e-9)

0404 |         if SERVO_SLEW_DPS > 0 and dt > 0:

0405 |             max_step = SERVO_SLEW_DPS * dt

0406 |             diff = target - self._servo_last_angle

0407 |             if abs(diff) > max_step:

0408 |                 target = self._servo_last_angle + (max_step if diff > 0 else -max_step)

0409 |

0410 |         # 3) remember for next call

0411 |         self._servo_last_angle = target

0412 |         self._servo_last_ns = now_ns

0413 |

0414 |         # 4) convert to pulse & send (unchanged)

0415 |         pulse = int(SERVO_PULSE_MIN + (SERVO_PULSE_MAX - SERVO_PULSE_MIN) *

0416 |                     ((target - SERVO_MIN_ANGLE) / (SERVO_MAX_ANGLE - SERVO_MIN_ANGLE)))

0417 |         self.pca.channels[SERVO_CHANNEL].duty_cycle = int(pulse * 65535 / SERVO_PERIOD)

0418 |

0419 |         return target  # handy if you want to log the actual command

0420 |

0421 |     def rotate_motor(self, speed):

0422 |         duty_cycle = int(min(max(abs(speed), 0), 100)/100*65535)

0423 |         if speed >= 0:

0424 |             self.pca.channels[MOTOR_FWD].duty_cycle = duty_cycle

0425 |             self.pca.channels[MOTOR_REV].duty_cycle = 0

0426 |         else:

0427 |             self.pca.channels[MOTOR_FWD].duty_cycle = 0

0428 |             self.pca.channels[MOTOR_REV].duty_cycle = duty_cycle

0429 |

0430 |     def stop_motor(self):

0431 |         self.pca.channels[MOTOR_FWD].duty_cycle = 0

0432 |         self.pca.channels[MOTOR_REV].duty_cycle = 0

0433 |

0434 |     def set_state_speed(self, state):

0435 |         # Set motor speed based on FSM state.

0436 |         #speed = STATE_SPEED.get(state, SPEED_CRUISE)  # default to cruise if unknown

0437 |         speed = state_speed_value(state)

0438 |         self.rotate_motor(speed)

0439 |

0440 |     def get_distance(self, sensor):

0441 |         #sensor: a gpiozero.DistanceSensor instance

0442 |         try:

0443 |             d = sensor.distance  # returns meters

0444 |             if d is None:

0445 |                 return None

0446 |             return d * 100  # convert to cm

0447 |         except:

0448 |             return None

0449 |

0450 |     def stable_filter(self, new_val, prev_val, alpha=FILTER_ALPHA, max_jump=FILTER_JUMP):

0451 |     #Reject large spikes and apply exponential moving average smoothing.

0452 |         if new_val is None:

0453 |             return prev_val  # keep previous if invalid

0454 |         if prev_val is not None and abs(new_val - prev_val) > max_jump:

0455 |             new_val = prev_val  # reject spike

0456 |         if prev_val is None:

0457 |             return new_val

0458 |         return alpha * new_val + (1 - alpha) * prev_val

0459 |

0460 |     def filtered_distance(self, sensor_obj, history, smooth_attr, sensor_type='us'):

0461 |          try:

0462 |              d = sensor_obj.range / 10.0 if sensor_type == 'tof' else sensor_obj.distance * 100.0

0463 |          except:

0464 |              d = None

0465 |

0466 |          history.append(d)

0467 |          valid = [x for x in history if x is not None]

0468 |          if not valid:

0469 |              return 999

0470 |

0471 |          median_val = np.median(valid)

0472 |          avg_val = np.mean(valid)

0473 |          #filtered_val = 0.7 * median_val + 0.3 * avg_val

0474 |          filtered_val = median_val

0475 |

0476 |          prev_val = getattr(self, smooth_attr)

0477 |          alpha = FILTER_ALPHA_TOF if sensor_type == 'tof' else FILTER_ALPHA

0478 |          max_jump = FILTER_JUMP_TOF if sensor_type == 'tof' else FILTER_JUMP

0479 |          smoothed_val = self.stable_filter(filtered_val, prev_val, alpha, max_jump)

0480 |

0481 |          setattr(self, smooth_attr, smoothed_val)

0482 |          return smoothed_val

0483 |

0484 |

0485 |     def safe_straight_control(self, d_left, d_right):

0486 |         correction = 0

0487 |         if d_left is not None and d_left < eff_soft_margin():

0488 |             correction = (eff_soft_margin() - d_left)*2

0489 |         elif d_right is not None and d_right < eff_soft_margin():

0490 |             correction = -(eff_soft_margin() - d_right)*2

0491 |         #correction = max(-MAX_CORRECTION, min(MAX_CORRECTION, correction))

0492 |         correction = max(-eff_max_correction(), min(eff_max_correction(), correction))

0493 |         #return SERVO_CENTER + correction

0494 |         angle = SERVO_CENTER + correction

0495 |         angle = max(SERVO_MIN_ANGLE, min(SERVO_MAX_ANGLE, angle))

0496 |         return angle

0497 |

0498 |     def turn_decision(self, d_left, d_right):

0499 |         left_open  = (d_left  is None) or (d_left  == 999) or (d_left  > TURN_DECISION_THRESHOLD)

0500 |         right_open = (d_right is None) or (d_right == 999) or (d_right > TURN_DECISION_THRESHOLD)

0501 |

0502 |         # Decide only when exactly one side is open; otherwise keep trying.

0503 |         if left_open ^ right_open:               #XOR: exactly one is True

0504 |             return "LEFT" if left_open else "RIGHT"

0505 |         return None # both open or both closed → keep trying

0506 |

0507 | robot = RobotController(pca)

0508 |

0509 | def sensor_reader():

0510 |     global sensor_data

0511 |     while True:

0512 |         # Left sensor

0513 |         if vl53_left:  # ToF

0514 |             left = robot.filtered_distance(vl53_left, robot.left_history, "smooth_left")

0515 |         elif us_left:  # ultrasonic

0516 |             left = robot.filtered_distance(us_left, robot.left_history, "smooth_left")

0517 |         else:

0518 |             left = None

0519 |

0520 |         # Right sensor

0521 |         if vl53_right:

0522 |             right = robot.filtered_distance(vl53_right, robot.right_history, "smooth_right")

0523 |         elif us_right:

0524 |             right = robot.filtered_distance(us_right, robot.right_history, "smooth_right")

0525 |         else:

0526 |             right = None

0527 |

0528 |         # Front sensor

0529 |         if vl53_front:

0530 |             front = robot.filtered_distance(vl53_front, robot.front_history, "smooth_front")

0531 |         elif us_front:

0532 |             front = robot.filtered_distance(us_front, robot.front_history, "smooth_front")

0533 |         else:

0534 |             front = None

0535 |

0536 |         # Save readings in global dictionary

0537 |         with sensor_lock:

0538 |             sensor_data["front"] = front

0539 |             sensor_data["left"] = left

0540 |             sensor_data["right"] = right

0541 |

0542 |         #time.sleep(SENSOR_DELAY)

0543 |         sensor_tick.wait(SENSOR_DELAY)

0544 |         sensor_tick.clear()

0545 |

0546 | # ===============================

0547 | # FINITE STATE MACHINE: robot_loop

0548 | # ===============================

0549 | class RobotState(Enum):

0550 |     IDLE = auto()

0551 |     CRUISE = auto()

0552 |     TURN_INIT = auto()

0553 |     TURNING = auto()

0554 |     POST_TURN = auto()

0555 |     STOPPED = auto()

0556 |

0557 | # Keep a global locked_turn_direction variable to persist across runs (reset in stop_loop)

0558 | locked_turn_direction = None

0559 |

0560 | def robot_loop():

0561 |

0562 |     #global loop_flag, readings_flag, status_text, turn_count, lap_count, locked_turn_direction

0563 |     global status_text, turn_count, lap_count, locked_turn_direction, stop_reason, obstacle_wait_deadline

0564 |     state = RobotState.IDLE

0565 |     direction = None

0566 |     yaw = 0.0

0567 |     turn_start_yaw = 0.0

0568 |     turn_start_time = 0.0

0569 |     post_turn_start = 0.0

0570 |     #last_turn_time = -TURN_LOCKOUT

0571 |     last_turn_time = -999

0572 |     #last_time = time.monotonic()

0573 |     #start_time = time.monotonic()

0574 |     last_ns = time.monotonic_ns()

0575 |     start_ns = last_ns

0576 |     use_post_turn_ref = False

0577 |     post_turn_ref_diff = None

0578 |     correction_active = False

0579 |     correction_start_time = 0.0

0580 |     correction_angle = SERVO_CENTER

0581 |     current_servo_angle = SERVO_CENTER

0582 |

0583 |     # Ensure robot stopped at start

0584 |     robot.stop_motor()

0585 |     robot.set_servo(SERVO_CENTER)

0586 |

0587 |     while True:

0588 |         #current_time = time.monotonic()

0589 |         #dt = current_time - last_time

0590 |         #last_time = current_time

0591 |         current_ns = time.monotonic_ns()

0592 |         dt = (current_ns - last_ns) * 1e-9   # seconds

0593 |         last_ns = current_ns

0594 |         current_time = current_ns * 1e-9     # float seconds, used by UI/timers

0595 |

0596 |         #if not readings_flag or not loop_flag:

0597 |         #if not readings_flag:

0598 |         if not readings_event.is_set():

0599 |             # if readings not started, keep motors stopped and idle

0600 |             robot.stop_motor()

0601 |             state = RobotState.IDLE

0602 |             status_text = "Idle"

0603 |             #time.sleep(LOOP_DELAY)

0604 |             sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0605 |             continue

0606 |

0607 |         # ----------------------------

0608 |         # Sensor reads & gyro integration (always update while readings_flag)

0609 |         # ----------------------------

0610 |         with sensor_lock:

0611 |             d_front = sensor_data["front"]

0612 |             d_left = sensor_data["left"]

0613 |             d_right = sensor_data["right"]

0614 |

0615 |         # Update robot attributes for plotting/logging and decision making

0616 |         robot.d_front = d_front

0617 |         robot.d_left = d_left

0618 |         robot.d_right = d_right

0619 |

0620 |         # ---- Narrow corridor detector (sum of side distances) ----

0621 |         if not hasattr(robot_loop, "_narrow_mode"):

0622 |             robot_loop._narrow_mode = False

0623 |

0624 |         # Use None-safe math (ignore invalid readings)

0625 |         l = robot.d_left  if (robot.d_left  is not None and robot.d_left  != 999) else None

0626 |         r = robot.d_right if (robot.d_right is not None and robot.d_right != 999) else None

0627 |         sum_lr = None if (l is None or r is None) else (l + r)

0628 |

0629 |         enter_thresh = NARROW_SUM_THRESHOLD

0630 |         exit_thresh  = NARROW_SUM_THRESHOLD + NARROW_HYSTERESIS

0631 |

0632 |         prev_mode = robot_loop._narrow_mode

0633 |         if sum_lr is not None:

0634 |             if not robot_loop._narrow_mode and sum_lr < enter_thresh:

0635 |                 robot_loop._narrow_mode = True

0636 |             elif robot_loop._narrow_mode and sum_lr > exit_thresh:

0637 |                 robot_loop._narrow_mode = False

0638 |

0639 |         # Apply global factors

0640 |         global SPEED_ENV_FACTOR, DIST_ENV_FACTOR

0641 |         SPEED_ENV_FACTOR = NARROW_FACTOR_SPEED if robot_loop._narrow_mode else 1.0

0642 |         DIST_ENV_FACTOR  = NARROW_FACTOR_DIST  if robot_loop._narrow_mode else 1.0

0643 |

0644 |         # Optional console feedback

0645 |         if robot_loop._narrow_mode != prev_mode:

0646 |             mode_txt = "NARROW mode ON" if robot_loop._narrow_mode else "NARROW mode OFF"

0647 |             print(f"[corridor] {mode_txt} (L+R = {sum_lr:.1f} cm)" if sum_lr is not None else f"[corridor] {mode_txt}")

0648 |

0649 |         # Gyro integration

0650 |         #raw_gyro_z = mpu.gyro[2] - bias

0651 |         #ALPHA = 0.8

0652 |         #gyro_z_filtered = ALPHA * raw_gyro_z + (1 - ALPHA) * getattr(robot, 'gyro_z_prev', 0)

0653 |         #robot.gyro_z_prev = gyro_z_filtered

0654 |         #gyro_z_deg = gyro_z_filtered * (180 / 3.14159265)

0655 |         #yaw += gyro_z_deg * dt

0656 |

0657 |         raw_gyro_z = mpu.gyro[2] - bias      # rad/s from MPU6050

0658 |         ALPHA = 0.8

0659 |         gyro_z_filtered = ALPHA * raw_gyro_z + (1 - ALPHA) * getattr(robot, 'gyro_z_prev', 0.0)

0660 |         robot.gyro_z_prev = gyro_z_filtered

0661 |         yaw += (gyro_z_filtered * RAD2DEG) * dt

0662 |

0663 |

0664 |         # Append to deques for plotting/logging

0665 |         #elapsed_time = current_time - start_time

0666 |         elapsed_time = (current_ns - start_ns) * 1e-9

0667 |         time_data.append(elapsed_time)

0668 |         front_data.append(robot.d_front if robot.d_front is not None else 0)

0669 |         left_data.append(robot.d_left if robot.d_left is not None else 0)

0670 |         right_data.append(robot.d_right if robot.d_right is not None else 0)

0671 |         angle_data.append(yaw)

0672 |         state_data.append(state.name)

0673 |

0674 |         # If loop_flag is still False, do NOT run FSM/motors yet—just update plots

0675 |         #if not loop_flag:

0676 |         if not loop_event.is_set():

0677 |             robot.stop_motor()

0678 |             state = RobotState.IDLE

0679 |             status_text = "Sensor readings started"

0680 |             #time.sleep(LOOP_DELAY)

0681 |             sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0682 |             continue

0683 |

0684 |         # ----------------------------

0685 |         # FSM transitions & actions

0686 |         # ----------------------------

0687 |         if state == RobotState.IDLE:

0688 |             #status_text = "Ready (readings started, loop not started)" if readings_flag else "Idle"

0689 |             status_text = "Ready (readings started, loop not started)" if readings_event.is_set() else "Idle"

0690 |             robot.stop_motor()

0691 |             robot.set_servo(SERVO_CENTER)

0692 |             correction_active = False

0693 |             #if loop_flag:

0694 |             if loop_event.is_set():

0695 |                 state = RobotState.CRUISE

0696 |                 status_text = "Driving (cruise)"

0697 |                 correction_active = False  # Reset correction when transitioning to CRUISE

0698 |                 for speed in range(0, SPEED_CRUISE + 1):

0699 |                     #if not loop_flag:

0700 |                     if not loop_event.is_set():

0701 |                         robot.stop_motor()

0702 |                         break

0703 |                     robot.rotate_motor(speed)

0704 |                     #time.sleep(0.01)

0705 |                     sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0706 |

0707 |         elif state == RobotState.CRUISE:

0708 |             status_text = "Driving (cruise)"

0709 |

0710 |             # -------------------------------

0711 |             # Emergency stop: immediate

0712 |             # -------------------------------

0713 |             if robot.d_front is not None and robot.d_front < eff_stop_threshold():

0714 |                 robot.stop_motor()

0715 |                 robot.set_servo(SERVO_CENTER)

0716 |                 status_text = "Stopped! Obstacle ahead"

0717 |                 print(status_text)

0718 |                 state = RobotState.STOPPED

0719 |

0720 |                 stop_reason = "OBSTACLE"

0721 |                 obstacle_wait_deadline = current_time + OBSTACLE_WAIT_TIME

0722 |                 continue

0723 |

0724 |             # -------------------------------

0725 |             # Check if we can trigger a turn

0726 |             # -------------------------------

0727 |             front_triggered = (robot.d_front is not None and robot.d_front < eff_front_turn_trigger())

0728 |             lockout_ok = (current_time - last_turn_time >= TURN_LOCKOUT)

0729 |

0730 |             if front_triggered and lockout_ok and loop_event.is_set():

0731 |                 # immediately enter TURN_INIT and drop speed

0732 |                 state = RobotState.TURN_INIT

0733 |                 status_text = "Approaching turn – waiting for side to open"

0734 |                 continue

0735 |

0736 |             # -------------------------------

0737 |             # Normal cruise servo control

0738 |             # Safe straight control active only after the 1st turn

0739 |             # -------------------------------

0740 |             if turn_count >= 1:

0741 |             # Wall-following correction after the first turn

0742 |             desired_servo_angle = robot.safe_straight_control(robot.d_left, robot.d_right)

0743 |

0744 |                 # Start correction if robot is too close to a wall

0745 |                 if (robot.d_left is not None and robot.d_left < eff_soft_margin()) or (robot.d_right is not None and robot.d_right < eff_soft_margin()):

0746 |                     if not correction_active:  # Start correction if not already active

0747 |                         correction_active = True

0748 |                         correction_start_time = current_time  # Record the start time of correction

0749 |                         status_text = "Driving (correction)"

0750 |                         print(f"↔️ Correction started. Servo angle: {desired_servo_angle}")

0751 |

0752 |                 # If correction is active, apply the correction angle

0753 |                 if correction_active:

0754 |                     # If correction duration is up, stop correction

0755 |                     if current_time - correction_start_time >= CORRECTION_DURATION:

0756 |                         correction_active = False

0757 |                         desired_servo_angle = SERVO_CENTER  # Reset to center once correction is done

0758 |                         status_text = "Driving (correction ended)"

0759 |                         print(f"↔️ Correction ended. Servo angle: {desired_servo_angle}")

0760 |

0761 |                 robot.set_servo(desired_servo_angle)  # Apply the desired servo angle (corrected or not)

0762 |

0763 |             else:

0764 |                 # Before the first turn, keep the servo centered and don't apply correction

0765 |                 desired_servo_angle = SERVO_CENTER

0766 |                 correction_active = False

0767 |                 robot.set_servo(desired_servo_angle)  # Keep centered

0768 |

0769 |             # -------------------------------

0770 |             # Timed servo correction behavior

0771 |             # -------------------------------

0772 |             if correction_active:

0773 |                 if current_time - correction_start_time >= CORRECTION_DURATION:

0774 |                     # Re-evaluate desired angle; if still not center, renew correction

0775 |                     if desired_servo_angle != SERVO_CENTER:

0776 |                         correction_start_time = current_time

0777 |                         correction_angle = desired_servo_angle

0778 |                         robot.set_servo(correction_angle)

0779 |                         status_text = "Driving (renewed correction)"

0780 |                         print(f"↔️ Straight correction renewed. Servo angle: {correction_angle}")

0781 |                     else:

0782 |                         correction_active = False

0783 |

0784 |             #robot.rotate_motor(SPEED_CRUISE)

0785 |             robot.set_state_speed(state.name)

0786 |

0787 |         elif state == RobotState.TURN_INIT:

0788 |             # Stay slow and keep wheels mostly straight while we wait for a side to open.

0789 |             # If you prefer slight wall-following here, use safe_straight_control().

0790 |             status_text = "Turn init – waiting for open side"

0791 |             robot.set_state_speed(state.name)

0792 |             correction_active = False  # Reset when entering TURN_INIT

0793 |

0794 |             # fail-safe: if front becomes safe again, return to cruise

0795 |             if robot.d_front is not None and robot.d_front >= eff_front_turn_trigger():

0796 |                 robot.set_servo(SERVO_CENTER)

0797 |                 state = RobotState.CRUISE

0798 |                 status_text = "Driving (cruise)"

0799 |                 # brief yield

0800 |                 sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0801 |                 continue

0802 |

0803 |             # keep the wheels centered (or gentle straight correction)

0804 |             #desired_servo_angle = robot.safe_straight_control(robot.d_left, robot.d_right) if turn_count >= 1 else SERVO_CENTER

0805 |             #robot.set_servo(desired_servo_angle)

0806 |             if turn_count >= 1:

0807 |                  desired_servo_angle = robot.safe_straight_control(robot.d_left, robot.d_right)

0808 |             else:

0809 |                  # No correction before first turn: keep the wheels centered (or gentle straight correction)

0810 |                  desired_servo_angle = SERVO_CENTER

0811 |             robot.set_servo(desired_servo_angle)

0812 |

0813 |             # Decide direction only when exactly one side is open

0814 |             proposed_direction = robot.turn_decision(robot.d_left, robot.d_right)

0815 |

0816 |             # If neither or both are open, keep waiting at TURN_INIT speed

0817 |             if proposed_direction is None or proposed_direction == 999:

0818 |                 sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0819 |                 continue

0820 |

0821 |             # Turn lock mechanic (unchanged logic, just applied here)

0822 |             if LOCK_TURN_DIRECTION == 1:

0823 |                 if locked_turn_direction is None:

0824 |                     locked_turn_direction = proposed_direction

0825 |                     print(f"Turn direction locked to {locked_turn_direction}")

0826 |                 elif proposed_direction != locked_turn_direction:

0827 |                     # keep waiting at TURN_INIT speed for the locked direction to open

0828 |                     status_text = f"Ignoring {proposed_direction} (locked to {locked_turn_direction})"

0829 |                     robot.set_servo(desired_servo_angle)

0830 |                     sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0831 |                     continue

0832 |                 direction = locked_turn_direction

0833 |             else:

0834 |                 direction = proposed_direction

0835 |

0836 |             # Commit the turn now that one side is open and lock (if any) allows it

0837 |             print(f"🔄 Turn initiated {direction}. Left: {robot.d_left if robot.d_left is not None else -1:.1f} cm, Right: {robot.d_right if robot.d_right is not None else -1:.1f} cm")

0838 |             turn_start_yaw = yaw

0839 |             turn_start_time = current_time

0840 |             angle = TURN_ANGLE_LEFT if direction == "LEFT" else TURN_ANGLE_RIGHT

0841 |             robot.set_servo(angle)

0842 |             robot.set_state_speed("TURNING")

0843 |             state = RobotState.TURNING

0844 |

0845 |         elif state == RobotState.TURNING:

0846 |             # active turning: monitor yaw & safety/time conditions to stop

0847 |             # compute turn angle relative to start

0848 |             turn_angle = yaw - turn_start_yaw  # positive for LEFT, negative for RIGHT

0849 |             target_angle = TARGET_TURN_ANGLE if direction == "LEFT" else -TARGET_TURN_ANGLE

0850 |             robot.set_state_speed("TURNING")

0851 |             stop_condition = False

0852 |

0853 |             # Condition A: side distance convergence (monitor opposite wall)

0854 |             #side_distance_monitor = robot.d_left if direction == "RIGHT" else robot.d_right

0855 |             #MIN_SIDE_CHANGE = 5  # cm

0856 |             #if side_distance_monitor is not None and turn_reference_side_distance is not None:

0857 |             #    if abs(side_distance_monitor - turn_reference_side_distance) > MIN_SIDE_CHANGE:

0858 |             #        lower_bound = turn_reference_side_distance * 0.95

0859 |             #        upper_bound = turn_reference_side_distance * 1.05

0860 |             #        if lower_bound <= side_distance_monitor <= upper_bound:

0861 |             #            print("Stop Turn - Opposite distance")

0862 |             #            stop_condition = True

0863 |

0864 |             # Condition B: front + side safe

0865 |             #side_distance = robot.d_left if direction == "LEFT" else robot.d_right

0866 |             #if robot.d_front is not None and robot.d_front > FRONT_SAFE_DISTANCE and side_distance is not None and side_distance > SIDE_SAFE_DISTANCE:

0867 |             #    print("Stop Turn - Front distance")

0868 |             #    stop_condition = True

0869 |

0870 |             # Condition C: reached target angle ± tolerance

0871 |             if abs(turn_angle - target_angle) <= TURN_ANGLE_TOLERANCE:

0872 |                 print("Stop Turn - Target Angle")

0873 |                 stop_condition = True

0874 |

0875 |             # Condition D: timeout or extreme yaw

0876 |             if current_time - turn_start_time > TURN_TIMEOUT:

0877 |                 print("Stop Turn - Max turn time")

0878 |                 stop_condition = True

0879 |             if abs(turn_angle) >= MAX_TURN_ANGLE:

0880 |                 print("Stop Turn - Max turn angle")

0881 |                 stop_condition = True

0882 |

0883 |             # If any stop condition met -> finish turn

0884 |             if stop_condition:

0885 |                 robot.stop_motor()

0886 |                 robot.set_servo(SERVO_CENTER)

0887 |                 current_servo_angle = SERVO_CENTER

0888 |                 last_turn_time = current_time

0889 |                 # Snap yaw to target for consistency

0890 |                 yaw = turn_start_yaw + target_angle

0891 |                 # update counters

0892 |                 turn_count += 1

0893 |                 if turn_count % 4 == 0:

0894 |                     lap_count += 1

0895 |                     print(f"✅ Lap completed! Total laps: {lap_count}")

0896 |                     # if reached max laps -> do final drive then stop (as in original logic)

0897 |                     if MAX_LAPS > 0 and lap_count >= MAX_LAPS:

0898 |                         # Drive forward a little before stopping

0899 |                         robot.set_servo(SERVO_CENTER)

0900 |                         robot.set_state_speed(state.name)

0901 |                         #time.sleep(POST_LAP_DURATION)

0902 |                         sensor_tick.wait(POST_LAP_DURATION); sensor_tick.clear()

0903 |                         robot.stop_motor()

0904 |                         stop_reason = "LAPS"

0905 |                         #loop_flag = False

0906 |                         #state = RobotState.STOPPED

0907 |                         #status_text = f"Stopped - Max Laps ({MAX_LAPS}) reached"

0908 |                         loop_event.clear()

0909 |                         sensor_tick.set()  # wake waits immediately

0910 |                         state = RobotState.STOPPED

0911 |                         status_text = f"Stopped - Max Laps ({MAX_LAPS}) reached"

0912 |                         continue

0913 |

0914 |                 # prepare post-turn

0915 |                 post_turn_start = current_time

0916 |                 state = RobotState.POST_TURN

0917 |                 status_text = "Driving (post-turn)"

0918 |

0919 |         elif state == RobotState.POST_TURN:

0920 |             # drive straight for short duration then return to CRUISE

0921 |             if current_time - post_turn_start < POST_TURN_DURATION:

0922 |                 robot.set_servo(SERVO_CENTER)

0923 |                 robot.set_state_speed(state.name)

0924 |                 status_text = "Driving (post-turn)"

0925 |             else:

0926 |                 state = RobotState.CRUISE

0927 |                 status_text = "Driving (cruise)"

0928 |

0929 |         #elif state == RobotState.STOPPED:

0930 |             # motors must be stopped; remain stopped until loop_flag cleared by GUI or user

0931 |         elif state == RobotState.STOPPED:

0932 |             robot.stop_motor()

0933 |             robot.set_servo(SERVO_CENTER)

0934 |

0935 |             # --- Obstacle-driven stop: auto-retry after OBSTACLE_WAIT_TIME ---

0936 |             if stop_reason == "OBSTACLE":

0937 |                 # nice status with countdown

0938 |                 remaining = max(0.0, obstacle_wait_deadline - current_time)

0939 |                 status_text = f"Stopped (obstacle) – retry in {remaining:.1f}s"

0940 |

0941 |                 # time to retry? re-check front distance

0942 |                 if current_time >= obstacle_wait_deadline:

0943 |                     # quick re-sample already happens in the sensor thread;

0944 |                     # just use the latest filtered reading here:

0945 |                     d_front_now = robot.d_front

0946 |

0947 |                     if d_front_now is not None and d_front_now >= eff_stop_threshold():

0948 |                         # clear → resume cruise

0949 |                         print("✅ Obstacle cleared — resuming")

0950 |                         status_text = "Driving (cruise)"

0951 |                         stop_reason = None

0952 |                         state = RobotState.CRUISE

0953 |                         robot.set_state_speed("CRUISE")

0954 |                         # small fall-through; next loop iteration will continue normally

0955 |                     else:

0956 |                         # still blocked → schedule another check in OBSTACLE_WAIT_TIME

0957 |                         obstacle_wait_deadline = current_time + OBSTACLE_WAIT_TIME

0958 |

0959 |                 # yield CPU while waiting

0960 |                 sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0961 |                 continue

0962 |

0963 |             # --- User or laps stop: stay stopped until user action (original behavior) ---

0964 |             status_text = "Stopped"

0965 |             sensor_tick.wait(LOOP_DELAY); sensor_tick.clear()

0966 |             continue

0967 |

0968 | # ===============================

0969 | # GUI SECTION

0970 | # ===============================

0971 | def launch_gui():

0972 |     """Initialize Tkinter GUI, matplotlib plots, sliders, and status indicators."""

0973 |     global btn_readings, btn_start, btn_stop, root, canvas

0974 |     global ax_front, ax_side, ax_angle

0975 |     global front_line, left_line, right_line, angle_line

0976 |     global status_circle, status_canvas, status_text_id, lbl_status

0977 |     global lbl_turns, lbl_laps

0978 |     global sliders_frame, slider_vars, btn_export

0979 |

0980 |     root = tk.Tk()

0981 |     root.title("VivaLaVida Robot Control")

0982 |

0983 |     slider_vars = {}

0984 |

0985 |     # ---------- Matplotlib Figure ----------

0986 |     fig, (ax_front, ax_side, ax_angle) = plt.subplots(3, 1, figsize=(6, 8))

0987 |     fig.tight_layout(pad=3.0)

0988 |

0989 |     ax_front.set_ylim(0, 350)

0990 |     ax_front.set_title("Front Sensor")

0991 |     ax_front.grid(True)

0992 |     front_line, = ax_front.plot([], [], color="blue")

0993 |

0994 |     ax_side.set_ylim(-150, 150)

0995 |     ax_side.set_title("Left (+Y) vs Right (-Y) Sensors")

0996 |     ax_side.grid(True)

0997 |     ax_side.axhline(0, color="black")

0998 |     left_line, = ax_side.plot([], [], color="green")

0999 |     right_line, = ax_side.plot([], [], color="orange")

1000 |

1001 |     ax_angle.set_ylim(-180, 180)

1002 |     ax_angle.set_title("Yaw Angle")

1003 |     ax_angle.grid(True)

1004 |     angle_line, = ax_angle.plot([], [], color="purple")

1005 |

1006 |     # ---------- Buttons ----------

1007 |     btn_readings = tk.Button(root, text="Start Readings", command=start_readings,

1008 |                              width=20, height=2, bg="blue", fg="white")

1009 |     btn_start = tk.Button(root, text="Start Loop", command=start_loop,

1010 |                           width=20, height=2, bg="green", fg="white", state="disabled")

1011 |     btn_stop = tk.Button(root, text="Stop Loop", command=stop_loop,

1012 |                          width=20, height=2, bg="red", fg="white", state="disabled")

1013 |

1014 |     # ---------- Status Labels ----------

1015 |     lbl_status = tk.Label(root, text="Idle", font=("Arial", 14))

1016 |     lbl_turns = tk.Label(root, text=f"Turns: {turn_count}", font=("Arial", 14))

1017 |     lbl_laps = tk.Label(root, text=f"Laps: {lap_count}", font=("Arial", 14))

1018 |

1019 |     # ---------- Circular Status Indicator ----------

1020 |     status_canvas = tk.Canvas(root, width=100, height=100, highlightthickness=0, bg=root.cget("bg"))

1021 |     status_circle = status_canvas.create_oval(10, 10, 90, 90, fill="grey", outline="")

1022 |     status_text_id = status_canvas.create_text(50, 50, text="IDLE", fill="white", font=("Arial", 14, "bold"))

1023 |

1024 |     # ---------- Sliders Frame ----------

1025 |     sliders_frame = tk.LabelFrame(root, text="Parameters", padx=10, pady=10)

1026 |     sliders_frame.grid(row=0, column=1, rowspan=8, sticky="ns", padx=10, pady=5)

1027 |

1028 |     # ---------- Column 3 frame for actions ----------

1029 |     actions_frame = tk.LabelFrame(root, text="Actions", padx=10, pady=10)

1030 |     actions_frame.grid(row=0, column=2, rowspan=8, sticky="ns", padx=10, pady=5)

1031 |

1032 |     # ---------- Two-column layout ----------

1033 |     btn_readings.grid(row=0, column=0, sticky="ew", padx=5, pady=2)

1034 |     btn_start.grid(row=1, column=0, sticky="ew", padx=5, pady=2)

1035 |     btn_stop.grid(row=2, column=0, sticky="ew", padx=5, pady=2)

1036 |     lbl_status.grid(row=3, column=0, pady=5)

1037 |     lbl_turns.grid(row=4, column=0, pady=2)

1038 |     lbl_laps.grid(row=5, column=0, pady=2)

1039 |     status_canvas.grid(row=6, column=0, pady=5)

1040 |

1041 |     canvas = FigureCanvasTkAgg(fig, master=root)

1042 |     canvas.get_tk_widget().grid(row=7, column=0, sticky="nsew", padx=5, pady=5)

1043 |

1044 |     root.grid_columnconfigure(0, weight=1)  # main buttons + plots

1045 |     root.grid_columnconfigure(1, weight=0)  # sliders

1046 |     root.grid_columnconfigure(2, weight=0)  # actions

1047 |     root.grid_rowconfigure(7, weight=1)     # plot row

1048 |

1049 |     # ---------- Slider Definitions ----------

1050 |     slider_groups = {

1051 |         "Speeds": [

1052 |             ("Cruise Speed", "SPEED_CRUISE", 0, 100, "int"),

1053 |             ("Turn Init Speed", "SPEED_TURN_INIT", 0, 100, "int"),

1054 |             ("Turn Speed", "SPEED_TURN", 0, 100, "int"),

1055 |             ("Post Turn Speed", "SPEED_POST_TURN", 0, 100, "int")

1056 |         ],

1057 |         "Driving & Safety, Wall following, Turns": [

1058 |             ("Turn angle Left", "TURN_ANGLE_LEFT", 55, 90, "int"),

1059 |             ("Turn angle Right", "TURN_ANGLE_RIGHT", 90, 125, "int"),

1060 |             ("Soft side Margin (cm)", "SOFT_MARGIN", 0, 75, "int"),

1061 |             ("Max Angle Correction dif at Soft Margin (o)", "MAX_CORRECTION", 0, 30, "int"),

1062 |             ("Stop Threshold (cm)", "STOP_THRESHOLD", 0, 100, "int"),

1063 |             ("Front Turn Trigger (cm)", "FRONT_TURN_TRIGGER", 50, 150, "int"),

1064 |             ("Turn Decision Threshold (Left/Right) (cm)", "TURN_DECISION_THRESHOLD", 0, 200, "int"),

1065 |             ("Turn Timeout (s)", "TURN_TIMEOUT", 0.1, 10, "float"),

1066 |             ("Max Turn Angle (o)", "MAX_TURN_ANGLE", 90, 120, "int"),

1067 |             ("Post Turn Duration (s)", "POST_TURN_DURATION", 0, 5, "float"),

1068 |             ("Turn Lockout (s)", "TURN_LOCKOUT", 0.1, 5, "float")

1069 |         ],

1070 |         "Other": [

1071 |             ("Sensor Filter N (Median)", "N_READINGS", 1, 10, "int"),

1072 |             ("Max Laps - 0 for infinite", "MAX_LAPS", 0, 20, "int"),

1073 |             ("Filter Smoothness (alpha)", "FILTER_ALPHA", 0.05, 0.9, "float"),

1074 |             ("Max Jump (cm)", "FILTER_JUMP", 5, 999, "int"),

1075 |         ]

1076 |     }

1077 |

1078 |     # ---------- Create Sliders ----------

1079 |     for group_name, sliders in slider_groups.items():

1080 |         group_frame = tk.LabelFrame(sliders_frame, text=group_name, padx=5, pady=5)

1081 |         group_frame.pack(fill="x", pady=5)

1082 |         for label_text, var_name, vmin, vmax, vartype in sliders:

1083 |             frame = tk.Frame(group_frame)

1084 |             frame.pack(fill="x", pady=2)

1085 |

1086 |             if vartype == "float":

1087 |                 var = tk.DoubleVar(value=globals()[var_name])

1088 |                 res = 0.1

1089 |             else:

1090 |                 var = tk.IntVar(value=globals()[var_name])

1091 |                 res = 1

1092 |

1093 |             slider_vars[var_name] = var

1094 |

1095 |             scale = tk.Scale(frame, from_=vmin, to=vmax, orient="horizontal",

1096 |                              variable=var, resolution=res)

1097 |             scale.pack(side="left", fill="x", expand=True)

1098 |

1099 |             lbl_val = tk.Label(frame, text=f"{label_text}: {var.get()}", width=25, anchor="w")

1100 |             lbl_val.pack(side="right", padx=5)

1101 |

1102 |             def make_callback(lbl, name, var, label_text):

1103 |                 def callback(value, label_text=label_text):

1104 |                     globals()[name] = var.get()

1105 |                     if name == "MAX_LAPS" and var.get() == 0:

1106 |                         lbl.config(text=f"{label_text}: ∞")

1107 |                     else:

1108 |                         lbl.config(text=f"{label_text}: {var.get():.1f}" if isinstance(var.get(), float) else f"{label_text}: {var.get()}")

1109 |                 return callback

1110 |

1111 |             scale.config(command=make_callback(lbl_val, var_name, var, label_text))

1112 |

1113 |     # ---------- Buttons under sliders ----------

1114 |     btn_export = tk.Button(actions_frame, text="Export CSV", command=export_data_csv,

1115 |                        width=20, height=2, bg="purple", fg="white")

1116 |     btn_export.pack(pady=10, fill="x")

1117 |

1118 |     btn_save_sliders = tk.Button(actions_frame, text="Save Sliders", command=save_sliders_json,

1119 |                              width=20, height=2, bg="green", fg="white")

1120 |     btn_save_sliders.pack(pady=5, fill="x")

1121 |

1122 |     btn_load_sliders = tk.Button(actions_frame, text="Load Sliders", command=load_sliders_json,

1123 |                              width=20, height=2, bg="blue", fg="white")

1124 |     btn_load_sliders.pack(pady=5, fill="x")

1125 |

1126 |     # ===============================

1127 |     # GUI Update Functions (nested)

1128 |     # ===============================

1129 |     def status_color(state: str) -> str:

1130 |         if "Stopped" in state:

1131 |             return "red"

1132 |         elif "Turning" in state:

1133 |             return "yellow"

1134 |         elif "Driving" in state:

1135 |             return "green"

1136 |         elif "Loop Started" in state or "Sensor readings started" in state:

1137 |             return "blue"

1138 |         else:

1139 |             return "grey"

1140 |

1141 |     def status_label(state: str) -> str:

1142 |         if "Stopped" in state:

1143 |             return "STOP"

1144 |         elif "Turning" in state:

1145 |             return "TURN"

1146 |         elif "Driving" in state:

1147 |             return "GO"

1148 |         elif "Loop Started" in state:

1149 |             return "LOOP"

1150 |         elif "Sensor readings started" in state:

1151 |             return "READ"

1152 |         else:

1153 |             return "IDLE"

1154 |

1155 |     def update_status():

1156 |         lbl_status.config(text=status_text)

1157 |         status_canvas.itemconfig(status_circle, fill=status_color(status_text))

1158 |         status_canvas.itemconfig(status_text_id, text=status_label(status_text))

1159 |         lbl_turns.config(text=f"Turns: {turn_count}")

1160 |         lbl_laps.config(text=f"Laps: {lap_count}" if slider_vars["MAX_LAPS"].get() > 0 else f"Laps: {lap_count}/∞")

1161 |         root.after(200, update_status)

1162 |

1163 |     _prev_label_pos = {"front": None, "left": None, "right": None, "angle": None}

1164 |     def update_plot():

1165 |         #global _prev_label_pos

1166 |

1167 |         if time_data:

1168 |             # Update lines

1169 |             front_line.set_data(range(len(front_data)), front_data)

1170 |             left_line.set_data(range(len(left_data)), left_data)

1171 |             right_line.set_data(range(len(right_data)), [-v for v in right_data])

1172 |             angle_line.set_data(range(len(angle_data)), angle_data)

1173 |

1174 |             # Set axis limits

1175 |             ax_front.set_xlim(0, MAX_POINTS)

1176 |             ax_side.set_xlim(0, MAX_POINTS)

1177 |             ax_angle.set_xlim(0, MAX_POINTS)

1178 |

1179 |             # Remove previous text annotations

1180 |             for ax in [ax_front, ax_side, ax_angle]:

1181 |                 for t in ax.texts:

1182 |                     t.remove()

1183 |

1184 |             # Helper for smoothing

1185 |             def smooth_move(prev, target, alpha=0.3):

1186 |                 if prev is None:

1187 |                     return target

1188 |                 return prev + alpha * (target - prev)

1189 |

1190 |             # ----------------------------

1191 |             # FRONT SENSOR

1192 |             # ----------------------------

1193 |             if len(front_data) > 0:

1194 |                 x = len(front_data) - 1

1195 |                 y_target = front_data[-1]

1196 |                 y_prev = _prev_label_pos["front"]

1197 |                 y_smooth = smooth_move(y_prev, y_target)

1198 |                 _prev_label_pos["front"] = y_smooth

1199 |                 ax_front.text(x, y_smooth, f"{y_target:.1f} cm", color="blue",

1200 |                               fontsize=9, fontweight="bold", va="bottom", ha="left")

1201 |

1202 |             # ----------------------------

1203 |             # LEFT SENSOR

1204 |             # ----------------------------

1205 |             if len(left_data) > 0:

1206 |                 x = len(left_data) - 1

1207 |                 y_target = left_data[-1]

1208 |                 y_prev = _prev_label_pos["left"]

1209 |                 y_smooth = smooth_move(y_prev, y_target)

1210 |                 _prev_label_pos["left"] = y_smooth

1211 |                 ax_side.text(x, y_smooth, f"L: {y_target:.1f} cm", color="green",

1212 |                              fontsize=9, fontweight="bold", va="bottom", ha="left")

1213 |

1214 |             # ----------------------------

1215 |             # RIGHT SENSOR

1216 |             # ----------------------------

1217 |             if len(right_data) > 0:

1218 |                 x = len(right_data) - 1

1219 |                 y_target = -right_data[-1]

1220 |                 y_prev = _prev_label_pos["right"]

1221 |                 y_smooth = smooth_move(y_prev, y_target)

1222 |                 _prev_label_pos["right"] = y_smooth

1223 |                 ax_side.text(x, y_smooth, f"R: {right_data[-1]:.1f} cm", color="orange",

1224 |                              fontsize=9, fontweight="bold", va="bottom", ha="left")

1225 |

1226 |             # ----------------------------

1227 |             # YAW ANGLE

1228 |             # ----------------------------

1229 |             if len(angle_data) > 0:

1230 |                 x = len(angle_data) - 1

1231 |                 y_target = angle_data[-1]

1232 |                 y_prev = _prev_label_pos["angle"]

1233 |                 y_smooth = smooth_move(y_prev, y_target)

1234 |                 _prev_label_pos["angle"] = y_smooth

1235 |                 ax_angle.text(x, y_smooth, f"{y_target:.1f}°", color="purple",

1236 |                               fontsize=9, fontweight="bold", va="bottom", ha="left")

1237 |

1238 |             # ----------------------------

1239 |             # Dynamic Y-axis scaling for yaw

1240 |             # ----------------------------

1241 |             if angle_data:

1242 |                 min_angle = min(angle_data)

1243 |                 max_angle = max(angle_data)

1244 |                 if max_angle - min_angle > 180:

1245 |                     ax_angle.set_ylim(-180, 180)

1246 |                 else:

1247 |                     ax_angle.set_ylim(min_angle - 10, max_angle + 10)

1248 |

1249 |             # Draw updated plots

1250 |             canvas.draw()

1251 |

1252 |         root.after(100, update_plot)

1253 |

1254 |     def on_closing():

1255 |         stop_loop()

1256 |         # Stop all ToF sensors cleanly

1257 |         for sensor in [vl53_left, vl53_right, vl53_front, vl53_back]:

1258 |             if sensor is not None:

1259 |                 try:

1260 |                     sensor.stop_continuous()

1261 |                 except Exception as e:

1262 |                     print(f"Warning: could not stop sensor {sensor}: {e}")

1263 |

1264 |         GPIO.cleanup()

1265 |         root.destroy()

1266 |

1267 |     root.protocol("WM_DELETE_WINDOW", on_closing)

1268 |     update_status()

1269 |     update_plot()

1270 |     root.mainloop()

1271 |

1272 | # -------------------------

1273 | # Start / Stop

1274 | # -------------------------

1275 |

1276 | def start_readings():

1277 |     #global readings_flag, status_text

1278 |     #readings_flag = True

1279 |     global status_text

1280 |     readings_event.set()

1281 |     status_text = "Sensor readings started"

1282 |     btn_readings.config(state="disabled")

1283 |     btn_start.config(state="normal")

1284 |

1285 |     # Start background sensor thread once

1286 |     if not hasattr(start_readings, "sensor_thread_started"):

1287 |         threading.Thread(target=sensor_reader, daemon=True).start()

1288 |         #threading.Thread(target=robot_loop, daemon=True).start()

1289 |         #start_readings.thread_started = True

1290 |         start_readings.sensor_thread_started = True

1291 |

1292 |     if not hasattr(start_readings, "loop_thread_started"):

1293 |         threading.Thread(target=robot_loop, daemon=True).start()

1294 |         start_readings.loop_thread_started = True

1295 |

1296 |

1297 | def start_loop():

1298 |     """Start the robot driving loop."""

1299 |     #global loop_flag, status_text

1300 |     global status_text

1301 |     #if not readings_flag:

1302 |     if not readings_event.is_set():

1303 |         print("Start sensor readings first!")

1304 |         return

1305 |

1306 |     #loop_flag = True

1307 |     loop_event.set()

1308 |     status_text = "🚗 Loop Started"

1309 |     btn_start.config(state="disabled")

1310 |     btn_stop.config(state="normal")

1311 |

1312 |     #if not hasattr(start_loop, "loop_thread_started"):

1313 |     #  threading.Thread(target=robot_loop, daemon=True).start()

1314 |     #  start_loop.loop_thread_started = True

1315 |

1316 | def stop_loop():

1317 |     """Stop the robot loop and motor."""

1318 |     #global loop_flag, status_text, turn_count, lap_count, locked_turn_direction

1319 |     #loop_flag = False

1320 |     global status_text, turn_count, lap_count, locked_turn_direction, stop_reason

1321 |     loop_event.clear()

1322 |     stop_reason = "USER"

1323 |     sensor_tick.set()  # wake any waits immediately

1324 |     robot.stop_motor()

1325 |     status_text = "Stopped"

1326 |     btn_start.config(state="normal")

1327 |     btn_stop.config(state="disabled")

1328 |

1329 |     # Reset counters

1330 |     turn_count = 0

1331 |     lap_count = 0

1332 |     lbl_turns.config(text=f"Turns: {turn_count}")

1333 |     lbl_laps.config(text=f"Laps: {lap_count}")

1334 |

1335 |     # Reset direction lock so a new session can re-choose

1336 |     locked_turn_direction = None

1337 |

1338 | # -------------------------

1339 | # Export to CSV

1340 | # -------------------------

1341 | def export_data_csv():

1342 |     filename = f"viva_data_{datetime.now().strftime('%Y%m%d_%H%M%S')}.csv"

1343 |     slider_values = {name: var.get() for name, var in slider_vars.items()}

1344 |

1345 |     with open(filename, mode='w', newline='') as file:

1346 |         writer = csv.writer(file)

1347 |         header = [

1348 |             "Time (s)", "Front (cm)", "Left (cm)", "Right (cm)", "Yaw (deg)",

1349 |             "State", "Turns", "Laps", "Servo Angle", "Speed", "Turning", "Direction"

1350 |         ]

1351 |         header += [name for name in slider_values.keys()]

1352 |         writer.writerow(header)

1353 |

1354 |         for i in range(len(time_data)):

1355 |             row = [

1356 |                 round(time_data[i], 2),

1357 |                 round(front_data[i], 2),

1358 |                 round(left_data[i], 2),

1359 |                 round(right_data[i], 2),

1360 |                 round(angle_data[i], 2),

1361 |                 state_data[i],

1362 |                 turn_count,

1363 |                 lap_count,

1364 |                 globals().get("servo_angle", 0),  # latest steering

1365 |                 globals().get("SPEED_CRUISE", 0),

1366 |                 "YES" if "Turning" in state_data[i] else "NO",

1367 |                 globals().get("locked_turn_direction", "")

1368 |             ]

1369 |             row += [slider_values[name] for name in slider_values.keys()]

1370 |             writer.writerow(row)

1371 |

1372 |     print(f"✅ Data exported to {filename}")

1373 |

1374 | # -------------------------

1375 | # Save/Load Slider Config

1376 | # -------------------------

1377 | def save_sliders_json():

1378 |     default_filename = f"sliders_config_{datetime.now().strftime('%Y%m%d')}.json"

1379 |     file_path = fd.asksaveasfilename(initialdir=BASE_DIR,

1380 |                                      initialfile=default_filename,

1381 |                                      defaultextension=".json",

1382 |                                      filetypes=[("JSON files", "*.json")],

1383 |                                      title="Save Slider Configuration")

1384 |     if file_path:

1385 |         data = {name: var.get() for name, var in slider_vars.items()}

1386 |         with open(file_path, "w") as f:

1387 |             json.dump(data, f, indent=4)

1388 |         print(f"Slider values saved to {file_path}")

1389 |

1390 | def load_sliders_json():

1391 |     file_path = fd.askopenfilename(initialdir=BASE_DIR,

1392 |                                    defaultextension=".json",

1393 |                                    filetypes=[("JSON files", "*.json")],

1394 |                                    title="Load Slider Configuration")

1395 |     if file_path:

1396 |         try:

1397 |             with open(file_path, "r") as f:

1398 |                 data = json.load(f)

1399 |             for name, value in data.items():

1400 |                 if name in slider_vars:

1401 |                     slider_vars[name].set(value)

1402 |                     globals()[name] = value

1403 |             print(f"Sliders restored from {file_path}")

1404 |         except Exception as e:

1405 |             print(f"Failed to load sliders: {e}")

1406 |

1407 | # ===============================

1408 | # MAIN

1409 | # ===============================

1410 |

1411 | if __name__ == "__main__":

1412 |     print(f"Starting VivaLaVida Autonomous Drive - GUI mode: {USE_GUI}")

1413 |

1414 |     if USE_GUI:

1415 |         launch_gui()

1416 |     else:

1417 |         # Start the sensor reading thread

1418 |         GPIO.output(RED_LED, GPIO.HIGH)

1419 |         GPIO.output(GREEN_LED, GPIO.LOW)

1420 |         sensor_thread = threading.Thread(target=sensor_reader, daemon=True)

1421 |         sensor_thread.start()

1422 |         print("Headless mode: waiting for START button...")

1423 |         GPIO.output(RED_LED, GPIO.LOW)     # turn off red when ready

1424 |         GPIO.output(GREEN_LED, GPIO.HIGH)  # green = ready to press the button

1425 |         try:

1426 |             # Wait for button press (GPIO input goes LOW when pressed)

1427 |             while GPIO.input(START_BTN) == 1:

1428 |                 time.sleep(0.05)

1429 |             print("✅ START button pressed! Beginning autonomous loop...")

1430 |             #readings_flag = True

1431 |             readings_event.set()

1432 |             time.sleep(2)  # wait 2 seconds before starting loop

1433 |             #loop_flag = True

1434 |             loop_event.set()

1435 |             print("Starting main robot loop...")

1436 |             # Start robot loop in this thread (blocking)

1437 |             robot_loop()

1438 |         except KeyboardInterrupt:

1439 |             print("

1440 | ❌ Keyboard interrupt received. Stopping robot loop.")

1441 |             #loop_flag = False

1442 |             loop_event.clear()

1443 |             sensor_tick.set()

1444 |             GPIO.cleanup()